# CDMA Q1110 DUMP Upgrade Guide (V1.0)

## 1. Power on and connect the handset to PC

If it is the first time to upgrade, the PC maybe pop the device installing window, please install the driver: unzip the package QSC1100usbdriver.rar and when the computer need install the driver of device, please select these file to install.

After driver installing, the port can be found in the “Device Manager” of PC, just as “USB Wireless Modem Diagnostics Interface (COM*)”.

## 2.  Run the Encrypt downloader (QSC1110 DownLoad Encrypt V1.0.0.exe)

- The flash size is 16 MB.

(2) Unzip the dump binary package to a folder which include APRG1110.hex, and xxxxx.bin; for instance, C381-Reliance_R_P1.01.02_S001_Lock.bin.

(3) Click “Browse…” button and select the folder that contains the binary files, mentioned in step (1);

(4) We should select the checkbox“Erase EFS”and keep the ESN&MEID and NV, please Select the CheckBox “Save QCN” and “Save ESN&MEID”;

(6) Select the “COM*” which is mentioned in step 1 in the first ListBox(the upper);

(7) Select “Download” in the second ListBox(the lower);

(8) Click “Start” button to start, and then wait for finishing the upgrade.

If you slelect the download mode in the second ListBox, move the mouse pointer to the ListBox again, it will pop the tip message of the selected download mode.

## 4. Note

1. You should upgrade the S/W version by the above steps.

2. The way of upgrading the binary is same as the way of upgrading the dump.

3. If you select “Erase EFS”, And the S/W version of upgrading is not dump. The phone will be white screen after finished upgrading. You need restore the QCN of this device, and reset the factory setting in the menu of settings.

4. If the S/W version is encrypted in the phone. You would be not allowed to download the normal S/W version. You only can upgrade the encrypted S/W version with the Encrypt download tool.

5，After downloading， we must factory reset the phone.