Normal SD Card Upgrade

1. Format micro SD card or delete its content.

2. Copy “update” folder into SD card.

3. Insert micro SD card, Power on the Phone, Input “##497613+Send” in idle mode, Select “SD Download”, and press “Yes”, Start upgrade the programs.

4. After upgrade complete, The phone will power off automatically.

Force SD Card Upgrade

1. Format micro SD card or delete its content.

2. Copy “update” folder into SD card.

3. Insert micro SD card, Power off the Phone, and then press “8+End” to Enter force upgrade.

4. After upgrade complete, The phone will power off automatically.