## LG-BD4000 Debug Screen Information

1. CDMA IDLE

2. CDMA CALL

0384   02222     13

2     1     -75    1x

-06.5    128      4.02

CHANNEL

SID

VOCODER

Protocol Revision

AC STATE

BATT

0384    02222      EV

Time      6:30

4    1    -095    1X

-10      216    3.84

CHANNEL

SID

VOCODER

CALL TIME

ACTIVE PILOT

RX POWER

Ec/Io

LNA Status

Active Pilot

Rx Power

Ec/Io

Protocol Revision

Rx Power

AC STATE

LNA Status

3. AMPS IDLE

0334   00003    ??

5    -086      A

3.99

0334   02222     ??

Time    6:30

- -086       A

3.99

CHANNEL

SID

VOCODER(NA)

MODE

RX POWER

BATT

AC STATE

4. AMPS CALL

CHANNEL

SID

VOCODER(NA)

CALL TIME

MODE

RX POWER

BATT

AC STATE

*SCREEN Item Description

A. CDMA IDLE

1 LINE

A. CHANNEL :  Frequency Channel on present (현재의 주파수 채널.)

B. SID : System ID Display (기지국 System ID 표시.)

C. VOCODER : EVRCEV, 13K13

2 LINE

A. AC STATE:   0  CDMA state

1  Sync channel state

2  Paging channel state

3  Traffic channel initialization state

4  Traffic channel state

5  CDMA exit state

B. LNA Status : Low Noise Amplifer Status : (0 – On,  1- Off.)

C. RX POWER :  Rx Signal Strength ( -50 ~ -120 dBM)

D. Protocol Revision : 1X ( P_REVr >= 6 ),  95 ( P_REVr < 6)

3 LINE

A. Ec/Io          :  간섭신호대 자기신호 비율(excluding AMPS)

B. ACTIVE PILOT : Active set pilot PN

C. BATT :  Battery Voltage (3.3V ~ 4.2V)

B. CDMA CALL

1 LINE

A. CHANNEL : Frequency Channel on present (현재의 주파수 채널.)

B. SID : System ID Display (기지국 System ID 표시.)

C. VOCODER : EVRCEV, 13K13

2 LINE

A. CALL TIME : Call time display during a call (seconds)
( 호접속이 일어나서부터 끊어질때가지의 시간(초단위) )

3 LINE

A. AC STATE : Same as above (위와 동일)

B. LNA Status : Low Noise Amplifier Status : (0 – On,  1- Off.)

C. RX POWER :  Rx Signal Strength ( -50 ~ -120 dBM)

D. Protocol Revision : 1X ( P_REVr >= 6 ),  95 ( P_REVr < 6)

4 LINE

A. Ec/Io          :  간섭신호대 자기신호 비율(excluding AMPS)

B. ACTIVE PILOT : Active set pilot PN

C. BATT :  Battery Voltage (3.3V ~ 4.2V)

C. AMPS IDLE

1 LINE

A. CHANNEL : Frequency Channel on present (현재의 주파수 채널.)

B. SID : System ID Display (기지국 System ID 표시.)

C. VOCODER :  ??로 표시.

2 LINE

A. AC STATE : Same as above (위와 동일)

B. RX POWER :  Rx Signal Strength ( -50 ~ -120 dBM)

C. RF MODE : AMPSA

3 LINE

A. BATT :  Battery Voltage (3.3V ~ 4.2V)

D. AMPS CALL

1 LINE

A. CHANNEL : Frequency Channel on present (현재의 주파수 채널.)

B. SID : System ID Display (기지국 System ID 표시.)

C. VOCODER : ??로 표시.

2 LINE

A. CALL TIME : Call time display during a call (seconds)
( 호접속이 일어나서부터 끊어질때가지의 시간(초단위) )

B. RF MODE : AMPSA

3 LINE

A. AC STATE : Same as above (위와 동일)

B. RX POWER :  Rx Signal Strength ( -50 ~ -120 dBM)

C. RF MODE : AMPSA

4 LINE

A. BATT :  Battery Voltage (3.3V ~ 4.2V)