# A500 Software Down-Load Process

- Preparation Elements

- Flash load(Down load program : latest ver. 37.9)

- Dongle(Program Hardware lock)

- JIG Box or Data Cable(PC link Cable)

- Procedure

- Run the Fastload program

- Setting

  - Down load control

    - Flash : Down load only main program

    - Flash + EEP : Down load both main and EEP data. In this case, calibration data will be disappeared.

  - Port Selection : Normaly COM1

  - Epower : Setting 1.5(+)

  - Serial Speed : Highest speed is 115,200bps

  - Number of phone : No. of phones can be download at one time

  - Start Up Mode : It can be used only select ‘Flash+EEP’ mode,

Setting ‘Normal Mode’ all the time

- Start-Up

  - Select the file on the Flash data and EEP data.

      - If select the ‘ Press button to synchronise eeprom directory with falsh directory’, directory of EEPROM will be same with Flash data

  - Press ‘ Start Loading Flash’

*JIG-BOX

    - Power Supply : setting 3.8~4.0V -> Run the program-> Turn on the Power supply

    - Battery : Battery setting -> Run the program -> press the power on Key

*Data Cable

    - Battery Setting -> Run the Program -> press the power on key

- Trouble Shooting

  - Connect the cable fitly

  - Setting ( Port, Epower, Serial Speed)

  - Enough Battery voltage

  - Connect the dongle fitly or dongle driver installation

* Please refer to ‘Fast load tool help’ for your detail information.