Software Update Manual

XELIBRI 2 and 4

Table of Contents:

XELIBRI	0

1. EQUIPMENT	2

2. DOWNLOAD	2

2.1 PROGRAM INSTALLATION	2

2.2. PROGRAM USE	3

3. CONFIGURATION	4

3.1. INTRODUCTION	4

3.2. PROGRAM INSTALLATION	5

3.3. PROGRAM USE	6

# 1. EQUIPMENT

Before the installation of the Download/configuration program check your equipment :

- Standard desktop PC. Pentium III / 128 MB  (with standard serial com)

- Windows NT environment

For the connection between PC and mobile use :

- Serial adapter (BootAdapter 2002 – see photo) .

- Connect the BootAdapter to power

- Connect the PC and BootAdapter with standard serial cable

- Use specific cable to connect mobile and BootAdapter.

# 2. DOWNLOAD

### 2.1 PROGRAM INSTALLATION

To install the soft, execute : setup_dwlpc.exe and follow instructions…

To launch the program : C:\Download_Siemens\dwlDev.exe

Following window appears :

### 2.2. PROGRAM USE

a) In Download type choose: Software ( it is the default value )

b) In Select a Project choose :

5087_RAM 16bits (X16,X16WAP,V21,WAB,X38,Z10) ( it is not default setting ! )

c) Then click on Select a file

and select the file.cla you want to download and wait few seconds

d) Select the Serial Port where your « Bootadapter 2002 » is connected

e) Insert a battery

And connect mobile with serial connector.

f) Press Start

Message TURN THE MOBILE ON appears, then press the On Key of the mobile.

Wait the end of the download ( 4 – 10 minutes ) then click OK.

g) For X2 product only:  A separate Package has to be downloaded !!!

After the *.cla download the Anim.bin package has to be added to the mobile

In Download type, choose: Package ( it is the default setting )

h) Then click on Select a package

and select the Anim.bin  to download ( wait few seconds )

Proceed with step e) and f) to start the download of the Package

# 3. CONFIGURATION

### 3.1. INTRODUCTION

The program « Service_Tool » allows to do a complete configuration of mobile without IMEI modification. This application permits to re-use mobile by deleting previous parameter and writing new parameters in product. This program is simple, operator plugs in product to serial connector, select a part number and launch the personalization.

### 3.2. PROGRAM INSTALLATION

To install “Service_Tool” program on your PC use the install kit delivery

Install Kit contents : setup.exe / service_.001 / service_.002

Installation:

- Run the Setup.exe file supply with the install kit.

Default installation path is "C:\service_tool_vx\"    (“vx” is the release version)

-	After installation the the install directory should have the following content:

### 3.3. PROGRAM USE

Because of software compatibility, a new “service_tool” program version is necessary for each new product software !.

a) Run the file « Service_Tool.exe », the program displays window on screen.

The program updates automatically the part number list you can use for the configuration .

In the left bottom corner the tool version and the mobile software version is shown.

Service Tool Version       Mobile software Version

b) Select the serial com number where BootAdapter is connected.

c) Select the part number corresponding to your product configuration.

Press Yes to confirm your choice, Selected part number appears on a window.

Blue LED indicates that program is ready to update mobile.

d) Insert a battery and connect mobile with serial connector.

e) Start the configuration by pressing the “START UPDATE” button.

A window “Turn the mobile On” appears:  Switch on mobile.

In case of start problem a new window appears.

Press OK, remove the battery and serial connector and do the test again.

If you meet the same problem

- 	check the battery charge

- check the mobile connector

- check the serial com number

If there is no problem, the configuration starts. (LED is blinking orange during update)

“TAT mode” on product LCD indicates that product is in “test autotest mode”

#### After some seconds the configuration is finished: Green LED indicates successful configuration

f) Unplug the serial connector, after some seconds the LED indicates blue again

another mobile can be updated now.