1. Your computer must first install Microsft ActiveSync4.2 or later. 
2. 如下图设置ActiveSync连接：右击电脑任务栏的ActiveSync小图标，连接设置，去掉“允许USB连接”。 2. Plans are set up ActiveSync to connect: Right-click the computer's taskbar icon ActiveSync small, to connect settings, remove the "Allow USB connection." 
 [attach]1206083[/attach][attach]1206084[/attach]

3. 手机关机，电脑插入USB线，另外一头插入手机的USB口。 3. Cell phone turned off, insert the USB computer line, and insert a cell phone's USB port. 
4. 同时按住手机左侧的“开关机键”和“后退键”，如下图： 4. At the same time, hold down the left side of the phone "switches key machine" and "back button", the following chart: 
 [attach]1206085[/attach]

5. 等待电脑自动安装驱动，当手机出现如下画面时，说明手机已进入DOWNLOAD模式，这时松开按键。 5. Wait for the computer to automatically install drivers, the following picture emerged when the mobile phone, cell phone that has entered the DOWNLOAD mode, then release the button. 
[attach]1206086[/attach]
6. 运行AmoiFlash.exe，选择软件目录，开始写软件。 6. Run AmoiFlash.exe, select the directory software, and began to write software. 如下图： Plans are as follows: 
 [attach]1206087[/attach]
7. 完成后手机自动开机。 7. Handset automatically after the completion of the boot. 

如果不成功，应该是电脑驱动没有安装好，重新开始上面的所有步骤，多试几次！ If unsuccessful, should not have to install a computer drive, start the steps above all, try a few more! 
实在不行，换一台没有装过ActiveSync的电脑（因为大家的windows是盗版的，ActiveSync使用久了会被windows盗版检测工具破坏）。 It will not do, not to replace computer equipment had ActiveSync (because the windows are pirated, ActiveSync will be used for a long time windows piracy tool to detect damage).

hardreset press and hold two "punction button" and then press "power" util screen show clear storge

end