| Product name | Security level |
| --- | --- |
| C300 | For internal use only |
| Product version | Total 45 pages |
| B106 |  |

            - HUAWEI CDMA Mobile Download Software Operation Guide

            - (For Internal Use Only）

| Drafted by: |  |  | Date: | yyyy-mm-dd |
| --- | --- | --- | --- | --- |
| Checked by: |  |  | Date: | yyyy-mm-dd |
| Checked by: |  |  | Date: | yyyy-mm-dd |
| Approved by: |  |  | Date: | yyyy-mm-dd |

            - Huawei Technologies Co., Ltd.

            - All rights reserved

            - Revision record

| - Date | - Revision Version | - CR ID / Defect ID | - Section Number | - Change Description | - Author |
| --- | --- | --- | --- | --- | --- |

            - Distribution List

| - Copy No. | - Holder's Name & Role | - Issue Date |
| --- | --- | --- |

Table of Contents

- Chapter 1 Software Description	1-1

- Chapter 2 Log in and Log out	2-1

- 2.1 Log in	2-1

- 2.2 Exit	2-1

- Chapter 3 Software Interface	3-1

- 3.1 Menu	3-1

- 3.1.2 [File] Menu	3-1

- 3.1.3 [View] Menu	3-1

- 3.1.4 [Tools] Menu	3-2

- 3.1.5 [Help] Menu	3-2

- 3.2 Tool Bar	3-2

- 3.3 Tool List	3-3

- 3.4 Main Window	3-4

- Chapter 4 System Configuration	4-1

- 4.1 Set login password	4-1

- 4.1.1 Setting login password	4-1

- 4.1.2 Modifying login password	4-2

- 4.1.3 Removing login password	4-2

- 4.2 Communication Configuration	4-3

- Chapter 5 Download Tool	5-1

- 5.1 Downloading APP Code through Serial Port	5-2

- 5.1.1 Normal Downloading	5-2

- 5.1.2 Forced Downloading	5-5

- 5.2 Downloading APP Code Using USB Port	5-8

- 5.3 Batch Download	5-8

- 5.4 Exporting Log	5-10

- Chapter 6 Read NV Items	6-1

- Chapter 7 Multi-COMs	7-1

- Chapter 8 Phonebook Tool	8-1

- 8.1 To open phonebook	8-1

- 8.2 To read phonebook	8-2

- 8.3 To write phonebook	8-2

- 8.4 To save phonebook	8-3

- Chapter 9 Prompts	9-1

- Chapter 10 System Configuration	10-1

# Software Description

            - HUAWEI Download Software is used for the maintenance of Huawei CDMA mobile stations. It runs on the Windows system.

            - The Download Software provides the following two downloading methods.

#### Normal Downloading

            - This method is applicable to the software upgrade when the mobile station is in normal power-on status.

Note：

            - For normal downloading, pay attention to the following restrictions:

- You cannot upgrade the software if the product id (see Figure 1-1) of the current software version of the mobile station is different from that of the software version to be upgraded and both numbers are not "0".

- If the current software version of the mobile station is B104 or later, you cannot fall back the software to versions earlier than B104.

- During normal downloading, do not replace the mobile station upon failure in powerdown of the mobile station caused by low battery charge level or PC power failure, or upon failure in normal downloading and startup of the mobile station caused by disconnected data cable. Forcibly download the APP to restore the previous back-up EFS file and NV data. Otherwise, you may lose the data of the mobile station and cannot use it properly.

          - Product ID

#### Forced Downloading

            - This method is applicable when the mobile station cannot be powered on normally.

Note：

            - For forced downloading, pay attention to the following restrictions:

- You cannot upgrade the software if the product id (see Figure 1-1) of the current software version of the mobile station is different from that of the software version to be upgraded and both numbers are not "0".

- If the current software version of the mobile station is B104 or later, you cannot fall back the software to versions earlier than B104.

- Regarding the software, you cannot back up or restore the EFS file and NV data.

- If you have to upgrade the software through forced downloading due to startup failure of the mobile station caused by powerdown or disconnected serial cable/USB cable during normal downloading, do not replace the mobile station. Otherwise, the data cannot be restored. After the APP code is downloaded, the software automatically restores the backup data during the previous normal downloading.

# Log in and Log out

            - Generally, the procedure to use the software is as follows:

##### Start and Log in to the software.

##### Perform operations.

##### Exit the software.

## Log in

            - When you start the software, the system displays the [Input Login Password] dialog box as shown in Figure 1-2 if you enable login password. To log in to the software, you have to enter the valid password. If you do not enable the login password, the system does not display the dialog box, and you can directly log in to the software.

          - [Input Login Password]

            - For details about how to set the password, refer to 4.1  Set login password.

Note：

A valid login password supports up to 16-digit characters.

## Exit

            - Select [File/Exit] to exit the software.

# Software Interface

          - Software Interface

## Menu

            - Figure 1-4 shows the main menu of the HUAWEI Download Software.

          - Main Menu

### [File] Menu

          - [File] Menu

            - [Exit] is used to close this software.

### [View] Menu

          - [View] Menu

            - There are two items on the [View] menu:

            - [Toolbar]: Used to open/close [Toolbar]

            - [Tool List]: Used to open/close [Tool List]

### [Tools] Menu

          - [Tools] Menu

            - There are two items on the [Tools] menu:

            - [SysConfig]: Used to display [System configuration] window

            - [Download Tool]: Used to display [Download Tool] window

            - [PhoneBook Tool]: Used to display [Phonebook Tool] window

### [Help] Menu

          - [Help] Menu

            - There are two items on the [Help] menu:

            - [Help Context]: Used to display the online help

            - [About the software]: Used to display the copyright information

            - [Convert to Chinese]: Used to switch between the two languages supported: English and simplified Chinese.

## Tool Bar

            - Figure 1-9 shows the tool bar.

          - Tool Bar

##### Click this button to display/hide the tool list.

##### Click this button to display the [SysConfig] window.

##### Click this button to switch between the two languages supported: English and simplified Chinese.

##### Click this button to display the online help.

## Tool List

            - The Tool List is located on the left of the main interface, as shown in Figure 1-10.

          - Tool List

            - You can find all the functions of the HUAWEI Download Software on the Tool List. You can display the operation window of a function by clicking the corresponding sub-node under the [Tool List] node.

#### System Configuration

            - This function includes Set Login Password and Communication Configuration.

- If you enable Set Login Password, you may set, modify, or remove the login password.

- If you enable Communication Configuration, you may set the port used for the communications between PC and mobile station.

#### Download Tool

            - This tool is used to download the APP code to a mobile station through a serial port or USB port.

#### Phonebook Tool

            - This tool is used to open local phonebook records and read, write, and save phonebook records of handsets.

## Main Window

            - Main Window is located on the right of the main interface. Normally, it is empty. After you select a function from the Tool List, Tool Bar, or Main Menu, the corresponding operation window will be displayed on the main window. Main Window can accommodate one operation window at a time.

# System Configuration

            - There are three ways to enable "System configuration":

##### Click [System Configuration] under the [Tool List] node.

##### Select [Tools/SysConfig]

##### Click

            - Figure 1-11 illustrates the SysConfig tool interface.

          - SysConfig tool interface

            - This function includes Set Login Password and Communication Configuration.

- If you enable Set Login Password, you may set, modify, or remove the login password.

- If you enable Communication Configuration, you may set the port used for the communications between PC and mobile station.

## Set login password

            - Login password refers to the one entered during software startup. During initial startup of the software, the "Enable login password" check box is by default selected. In this case, you have to enter the initial password to log in to the software. Through the "Set Login Password" function, you can set, modify, or remove the login password.

### Setting login password

            - Set the password as follows:

##### Select the check box "Enable login password" to display the [Set login password] window as shown in Figure 1-12.

          - Set login password

##### Enter your specified password in the "New password" box.

##### In the "Confirm password" box, enter the same password as input in step 2).

##### Click <OK> and the password will become enabled.

### Modifying login password

            - Modify the password as follows:

##### Click <Modify password> to display the window as shown in Figure 1-13.

          - Modify password

##### Enter the old login password in the "Old password" box.

##### Enter the new login password in the "New password" box.

##### Enter the same password as input in step 3) in the "Confirm password" box.

##### Click <OK> to make the new password take effect.

### Removing login password

            - Disable login password by referring to removing the login password. Remove the login password as follows:

##### Uncheck "Enable login password" to display the dialogue box as shown in Figure 1-14.

          - [Remove the password]

##### Enter the old login password in the "Old password" box.

##### Click <OK> to remove the login password.

## Communication Configuration

            - On the [Communication Configuration] window, to set the port number used for the communication between PC and mobile station, proceed as follows:

##### Set the port number.

##### Click <OK>.

            - The software supports the USB port. If the USB port is used, you need to install the USB driver and then set the communication port on the [System Configuration] window as follows:

##### Start the software.

##### Connect the mobile station and the PC using a USB cable.

##### If the mobile station is in the "OFF" state, press <Power> to switch on the mobile station.

##### The Windows system gives a prompt, indicating that new hardware has been detected and the corresponding driver shall be installed. Install the driver under the directory where the USB driver delivered with this software is located. You may skip this step if you have installed this driver before.

##### Right-click [My Computer] to select [Properties] on the shortcut menu.

##### Select the [Hardware] tab, and then click <Device Manager> to display the [Device Manager] window.

##### Click <Ports(COM&LPT)> to view the number of the port hosting Huawei CDMA mobile station.

##### Set the number of the port used for communications to the one displayed in step 7).

# Download Tool

            - There are two ways to enable "Download Tool":

##### Click [Download Tool] under the [Tool List] node

##### Select [Tools/Download Tool]

            - Figure 1-15 illustrates the download tool interface.

          - Download Tool Interface

            - This tool is used to download the APP code to a mobile station through a serial port or USB port.

Note

- Make sure that the battery of the mobile station is fully charged. Otherwise, insufficient power during the upgrade may lead to the upgrade failure.

- Do not disconnect the mobile station and the PC. Otherwise, upgrade failure may result.

## Downloading APP Code through Serial Port

            - When downloading the APP code through the serial port, you may select from the two methods below: normal downloading and forced downloading.

### Normal Downloading

            - If the mobile station operates normally, you can download the APP code using the normal downloading procedure, proceed as follows:

##### Start this software.

##### Click "System Configuration" on the "Tool List".

##### Set the port number. For details, refer to 4.2  Communication Configuration.

##### Click "Download Tool" on the "Tool List".

##### Click <Browse> and select the configuration file.

##### Click<Open> and the software displays the information about the parameter packet as shown in Figure 1-16.

          - parameter packet information

Note：

- Do not modify the configuration file or any file in the folder Configuration File. Otherwise, file verification will result in failure.

- B105 and later versions can display information about the parameter packet. Be sure to select the proper parameter packet before downloading.

##### Click <Download>.

            - The software automatically backs up the important data.

            - After the completion of the data backup, the mobile station is switched to the downloading mode and starts downloading the APP code as shown in Figure 1-17. The mobile station screen displays the message "Downloading indication".

          - Downloading APP code

            - After the APP code is downloaded, the mobile station restarts automatically. The download tool begins to restore the backup data and then add some files and data necessary for the APP code version as shown in Figure 1-18.

          - restore data and add some files and data

##### The downloading is completed.

          - downloading is completed

Caution：

- If errors such as power failure or cable disconnection occur during the downloading process and lead to the downloading failure, use the forced downloading instead. After the APP code is downloaded, the download tool begin to restore the backup data, you do not need to calibrate the RF parameters.

- No intervention is required for the whole downloading process. But if the data backup, restoration, or addition of resource files or data fails, a dialog box will appears, requiring you to select <Stop>, <Retry> or <Neglect> . It is recommended that you select <Retry>. If the operation fails after several retries, select <Neglect>. However, the mobile station may fail to operate normally. Do not select <Stop> unless necessary. Selecting <Stop> may result in malfunction of the mobile station.

### Forced Downloading

            - To download the APP code using the forced downloading method, proceed as follows:

##### Start this software.

##### Click "System Configuration" on the "Tool List".

##### Set the port number. For details, refer to 4.2  Communication Configuration.

##### Power off the mobile station.

##### Click "Download Tool" on the "Tool List".

##### Click<Browse> and select the configuration file.

Note：

Do not modify the configuration file or any file in the folder Configuration File. Otherwise, file verification will result in failure.

##### Press the numeric key "5" and "Power" key on the mobile station at the same time to enter the forced downloading status. And the mobile station screen displays the message "Forced downloading".

##### Click <Download> to start the forced downloading.

            - If the forced downloading is not caused by failure in normal downloading, Figure 1-20 is displayed. Click <Cancel> to stop the downloading. Otherwise, forced downloading is started.

          - Continue to force download?

            - After the APP code is downloaded, the Download Software will download some additional resource files and data required by the mobile station according to the configuration file.

            - The whole downloading process ends.

Caution：

- After the forced downloading, you need to calibrate the RF parameters before the mobile station can operate normally.

- During the Forced Downloading, you cannot back up or restore the user data or system data of the mobile station. Any attempt of data backup or restoration may lead to unavailability of some functions of the mobile station.

- If errors such as power failure or cable disconnection occur during the downloading process and lead to the downloading failure, repeat the forced downloading procedure

## Downloading APP Code Using USB Port

            - When downloading the APP code through the USB port, you may select from the two methods below: normal downloading and forced downloading.

            - The downloading procedure is the same as that through the serial port except the Communication Configuration.

## Batch Download

            - This software supports batch download. To download the configuration file, proceed as follows:

##### Start this software.

##### Click "System Configuration" on the "Tool List".

##### Set the port number.

##### Click "Download Tool" on the "Tool List".

##### Click <Browse> to select the configuration file.

##### Make sure the mobile station is in normal power-on status and is running normally with sound connection with PC.

##### Click <Batch Download>.

##### After you download the configuration file for one mobile station, Figure 1-21 is displayed.

          - try to connect a phone

            - During the downloading, all the buttons in the interface is disabled. After you finish downloading the configuration file for one mobile station, <Stop Batch Download> is activated and the software is ready for the same procedure for another mobile station. When you connect another mobile station in the normal power-on status, the software automatically starts the batch download procedure. You may also click <Stop Batch Download> to stop the batch download.

Note：

- Batch download is applicable to normal downloading instead of forced downloading.

- You can download the configuration file in batches either by using the serial port or by using the USB port.

## Exporting Log

  - This function is used to export the information in the message list to facilitate problem locating. Upon any problem during the downloading procedure, you may use this function to export the information to the file. Proceed as follows:

##### Click<Export Log>.

##### Select your path and specify the file name in the following dialog box.

##### Click <Save>.

# Read NV Items

- The Download Software can read NV items including MSIN、ESN (HEX) 、Software Version、Lock Code, the following figure demonstrates the process of reading NV items :

          - NV items reading

- Table 1-1 describes detailed information about the parameters to be read.

            - Parameters in NV items

| - Para. | - Description | - Value range | - Recom. value (default) | - Remarks |
| --- | --- | --- | --- | --- |
| - MSIN | - Mobile Station Identifier Number | - -- | - Allocated by CCITT | - Consisting of 10 decimal digits; |
| ESN | Electronic Serial Number | 32 binary digits | -- | provided by the mobile station manufacturer |
| Software Version | -- | -- | -- | Software version of the current mobile station in service |
| Lock Code | -- | 6 digits | “000000” | Lock phone so that only emergency dialing may be done |

            - Usage Guideline:

- Click <Read > to read the readable parameters in NV items from the mobile station.

# Multi-COMs

- The download software can download Application and Resource files with multiple COM ports simultaneously. In the system configuration dialog, The item Port number contains all the serial ports available. After the download software starts, The title of Mainframe and download dialog will display the serial port in use, you can select the serial port supported by the system to download the application and resource files.

          - COM port available

          - COM number in use

            - Usage Guideline:

- After you select serial port in item of Port number, system configuration dialog , the selected serial port number will be displayed on the title of mainframe and download dialog, and you can use this serial port to download the program into mobile terminal.

# Phonebook Tool

            - There are two ways to enable "Phonebook Tool":

##### Click [Phonebook Tool] under the [Tool List] item

##### Select [Tools/ Phonebook Tool]

            - Figure 1-15 illustrates the Phonebook Tool interface.

          - Phonebook Tool Interface

            - This tool is used to open the local phonebook file and read、write、save phonebook records of handsets.

Note：

- PST Tool supports the operation of phonebook records of mobile, UIM Card not supported by  PST Tool..

## To open local phonebook records

            - You can open the local phonebook file and display it using the phonebook tool, the steps as follows:

##### Start this software.

##### Click "Phonebook Tool" on the "Tool List".

##### Click <Open> and select the local phonebook file.

##### Click <Open> and the software displays all the records in the selected file as shown in Figure 1-16.

          - records information

## To read phonebook records

            - You can read the phonebook records of the handsets and display it using the phonebook tool, the steps as follows:

##### Start this software.

##### Click "Phonebook Tool" on the "Tool List".

##### Click <Read> and the software displays all the phonebook records of the handset as shown in Figure 1-162.

## To write phonebook records

            - You can edit and write the phonebook records into the handset using the phonebook tool, the steps as follows:

##### Start this software.

##### Click "Phonebook Tool" on the "Tool List".

##### Add, Delete, modify phonebook records.

##### Click <Write> and the software displays whether the phonebook records are correctly imported into the handset as shown in Figure 1-163.

          - records written into handset

## To save phonebook records

            - You can save the phonebook records which are currently displayed in the dialog, the steps as follows:

##### Start this software.

##### Click "Phonebook Tool" on the "Tool List".

##### Click <Save> and the software displays a dialog which indicates user to specify file name as shown in Figure 1-164.

          - The dialog to save records

##### Input file name and Click <Save> and the software displays a dialog which indicates user the operation is complete as shown in Figure 1-165.

          - To save records

# Prompts

            - During the above operations, the download software displays a number of prompts in the message list as shown in the table below to indicate the current operations or their corresponding operating status.

| No. | Prompt | Remarks |
| --- | --- | --- |
| 1 | Configuration file check sum failed | Contact the configuration file provider. |
| 2 | Configuration file check sum OK | -- |
| 3 | Backuping EFS files… | Includes the system file of the mobile station regarding phonebook and SMS; Depends on version. |
| 4 | Backuping Nv data… | Mainly includes key parameter of mobile station |
| 5 | Switching to download mode! | Prepares to download APP code |
| 6 | Trying to get ESN ... | This message is displayed after failure in getting the ESN of the mobile station. The software can attempt to get the ESN for up to ten times. |
| 7 | Could NOT get ESN! | Fails even after attempts for another ten times |
| 8 | ERROR: Mismatched ESN! | Probably caused by the replaced mobile station during the operation |
| 9 | ERROR: Invalid mobile phone parameter file, THIS PHONE MAY BE DESTROYED! | This message is displayed when the data fails to be backed up and the software is upgraded through forced downloading. In this case, the data is not restored after the APP is downloaded. |
| 10 | Force download! | Starts forced download anyway |
| 11 | Downloading file: | -- |
| 12 | Rebooting | Restarting mobile station |
| 13 | ERROR: Erase flash error! | Probably caused by power failure of the mobile station or loose connection of the data cable |
| 14 | Skip downloading application! | The APP file is not added to the configuration file. Data processing instead of software upgrade is conducted. |
| 15 | Creating EFS files Adding EFS file: … | -- |
| 16 | Restoring EFS files ... | -- |
| 17 | Restoring EFS files ... done! | -- |
| 18 | Restoring NV Data ... | Includes some system parameters of mobile station |
| 19 | Restoring NV Data ...done! | -- |
| 20 | ERROR: Open file …… | This file might not exist or is being used by other software. |
| 21 | ERROR: SPC code error! | The SPC code in the configuration file is not consistent with that of the mobile station. Contact the configuration file provider. |
| 22 | Trying write NV | -- |
| 23 | Connecting to board ... | -- |
| 24 | Unsupported degrade! | Depends on version |
| 25 | Found ERROR! | Stop the upgrade procedure upon any error found. |
| 26 | Done! | Do not disconnect the mobile station from the PC except when the upgrade failure is caused by the error found. |
| 27 | Reading phonebook file of the handset. | Exported phonebook file from the handset to PC |
| 28 | Writing phonebook of the handset. | Imported effective phonebook records currently displayed into the handset |
| 29 | Fails to export EFS files! - please check whether the mobile startups correctly! - please check whether the mobile is connected with PC! - please check whether the port is occupied by other program! | Exception during exporting phonebook file from the handset to PC |
| 30 | To write phonebook is ok ! | Complete to replace phonebook records of the handset with current records displayed in the PC software |
| 31 | To save record file: is Ok! | Complete to save phonebook records currently displayed as the local record CSV file |
| 32 | Idle State! | No operations for phonebook tool |
| 33 | Fails to open serial COM! - please check whether the port is occupied by other program! - please check whether the port is set correctly! | Exceptions that PC could not connect with the handset |

# System Configuration

            - Huawei CDMA Mobile Download Software runs in Windows98 or above systems and shall meet the following hardware requirements.

- Processor: Pentium II 233

- Memory: 64M

- Free space in hard disk: 100M

- Port: Serial port or USB port

- Display: resolution in the screen area: 800 x 600; 256 colors