# CDMA/GSM MOBILE CELLULAR PHONE

- LG-UW810

## SERVICE MANUAL

# Features of Mobile Subscriber Radio Handset

# (LG-UW810 Type)

#### 1. Frequency Scope

-  CELLULAR : 824.640~893.370MHz

-  US-PCS(CDMA) : 1850 ~ 1990 MHz

-  GSM900 : 880.2 MHz ~ 959.8 MHz

-  DCS1800 : 1710.2 MHz ~ 1879.8 MHz

-  PCS1900 : 1850 ~ 1990 MHz

-  GPS : 1574 ~ 1577 MHz

#### 2. Rated Output

-  CELLULAR : 0.224W

-  US-PCS(CDMA) : 0.224W

-  GSM900 : 2W

-  DCS1800 : 1W

-  PCS1900 : 1W

#### 3. Frequency Stability

-  CDMA  ＜ ±0.5PPM

 GSM/DCS1800/PCS1900＜ ±0.1PPM

# CHAPTER 1. System Introduction

## 1. System Introduction

## 1.1 CDMA Abstract

The CDMA mobile communication system has a channel hand-off function that is used for collecting the information on the locations and movements of mobile telephones from the cell site by automatically controlling several cell site through the setup of data transmission routes, and then enabling one switching system to carry out the automatic remote adjustment. This is to maintain continuously the call state through the automatic location confirmation and automatic radio channel conversion when the busy subscriber moves from the service area of one cell site to that of another by using automatic location confirmation and automatic radio channel conversion functions. The call state can be maintained continuously by the information exchange between switching systems when the busy subscriber moves from one Cellular system area to the other Cellular system area.

In the Cellular system, the cell site is a small-sized low output type and utilizes a frequency allocation system that considers mutual interference, in an effort to enable the re-use of corresponding frequency from a cell site separated more than a certain distance. The analog Cellular systems are classified further into an AMPS system, E-AMPS System, NMT system, ETACS system, and JTACS system depending on technologies used.

Unlike the time division multiple access (TDMA) or frequency division multiple access (FDMA) used in the band limited environment, the Code Division Multiple Access (CDMA) system which is one of digital Cellular systems is a multi-access technology under the interference limited environment. It can process more number of subscribers compared to other systems (TDMA system has the processing capacity three times greater than the existing FDMA system whereas CDMA system, about 12~15 times of that of the existing system).

CDMA system can be explained as follows; TDMA or CDMA can be used to enable each person to talk alternately or provide a separate room for each person when two persons desire to talk with each other at the same time, whereas FDMA can be used to enable one person to talk in soprano, whereas the other in bass (one of the two talkers can carry out synchronization for hearing in case there is a bandpass filter function in the area of the hearer). Another available method is to make two persons to sing in different languages at the same time, space, and frequency when wishing to let the audience hear the singing without being confused. This is the characteristic of CDMA.

On the other hand, when employing the CDMA technology, each signal has a different pseudo-random binary sequence used to spread the spectrum of carrier. A great number of CDMA signals share the same frequency spectrum. In the perspective of frequency area or time area, several CDMA signals are overlapped. Among these types of signals, only desired signal energy is selected and received through the use of pre-determined binary sequence; desired signals can be separated, and then received with the correlator used for recovering the spectrum into its original state. At this time, the spectrums of other signals that have different codes are not recovered into its original state, and appears as the self-interference of the system.

2. Features and Advantages of CDMA Mobile Phone

## 2.1 Various Types of Diversities

When employing the narrow band modulation (30kHz band) that is the same as the analog FM modulation system used in the existing Cellular system, the multi-paths of radio waves create a serious fading. However, in the CDMA broadband modulation (1.25MHz band), three types of diversities (time, frequency, and space) are used to reduce serious fading problems generated from radio channels in order to obtain high-quality calls.

Time diversity can be obtained through the use of code interleaving and error correction code whereas frequency diversity can be obtained by spreading signal energy to wider frequency band. The fading related to normal frequency can affect the normal 200~300kHz among signal bands and accordingly, serious effect can be avoided. Moreover, space diversity (also called path diversity) can be realized with the following three types of methods.

First, it can be obtained by the duplication of cell site receive antenna. Second, it can be obtained through the use of multi-signal processing device that receives a transmit signal having each different transmission delay time and then, combines them. Third, it can be obtained through the multiple cell site connection (Soft Handoff) that connects the mobile station with more than two cell sites at the same time.

## 2.2 Power Control

The CDMA system utilizes the forward (from a base station to mobile stations) and backward (from the mobile station to the base station) power control in order to increase the call processing capacity and obtain high-quality calls. In case the originating signals of mobile stations are received by the cell site in the minimum call quality level (signal to interference) through the use of transmit power control on all the mobile stations, the system capacity can be maximized.If the signal power of mobile station is received too strong, the performance of that mobile station is improved. However, because of this, the interference on other mobile stations using the same channel is increased and accordingly, the call quality of other subscribers is reduced unless the maximum accommodation capacity is reduced.

In the CDMA system, forward power control, backward open loop power control, and closed loop power control methods are used. The forward power control is carried out in the cell site to reduce the transmit power on mobile stations less affected by the multi-path fading and shadow phenomenon and the interference of other cell sites when the mobile station is not engaged in the call or is relatively nearer to the corresponding cell site. This is also used to provide additional power to mobile stations having high call error rates, located in bad reception areas or far away from the cell site.

The backward open loop power control is carried out in a corresponding mobile station; the mobile station measures power received from the cell site and then, reversely increases/decreases transmit power in order to compensate channel changes caused by the forward link path loss and terrain characteristics in relation to the mobile station in the cell site. By doing so, all the mobile transmit signals received by the base station have same strength.

Moreover, the backward closed loop power control used by the mobile station is performed to control power using the commands issued out by the cell site. The cell site receives the signal of each corresponding mobile station and compares this with the pre-set threshold value and then, issues out power increase/decrease commands to the corresponding mobile station every 1.25msec (800 times per second). By doing so, the gain tolerance and the different radio propagation loss on the forward/backward link are complemented.

## 2.3 Voice Encoder and Variable Data Speed

The bi-directional voice service having variable data speed provides voice communication which employs voice encoder algorithm having power variable data rate between the base station and the mobile station. On the other hand, the transmit voice encoder performs voice sampling and then, creates encoded voice packets to be sent out to the receive voice encoder, whereas the receive voice encoder demodulates the received voice packets into voice samples.

One of the two voice encoders described in the above is selected for use depending on inputted automatic conditions and message/data; both of them utilize four-stage frames of 9600, 4800, 2400, and 1200 bits per second for Cellular and 14400,7200,3600,1800 bits per second for PCS, so PCS provide relatively better voice quality (almost twice better than the existing celluar system). In addition, this type of variable voice encoder utilizes adaptive threshold values on selecting required data rate. It is adjusted in accordance with the size of background noise and the data rate is increased to high rate only when the voice of caller is inputted.

Therefore, background noise is suppressed and high-quality voice transmission is possible under the environment experiencing serious noise. In addition, in case the caller does not talk, data transmission rate is reduced so that the transmission is carried out in low energy. This will reduce the interference on other CDMA signals and as a result, improve system performance (capacity increased by about two times).

## 2.4 Protecting Call Confidentiality

Voice privercy is provided in the CDMA system by means of the private long code mask used for PN spreading. Voice privacy can ve applied on the traffic channels only. All calls are initiated using the public long code mask for PN spreading. The mobile station user may request voice privacy during call setup using the origination message or page response message, and during traffic channel operation using the long code transition request order.

The Transition to private long code mask will not be performed if authentication is not performed. To initiate a transition to the private or public long code mask, either the base station or the mobile station sends a long code transition request order on the traffic channel.

## 2.5 Soft Handoff

A handoff in which the mobile station commences communications with a new base station without interrupting communications with the old base station. Soft handoff can only be used between CDMA channels having identical freqeuncy assignments.

## 2.6 Frequency Re-Use and Sector Segmentation

Unlike the existing analog Cellular system, the CDMA system can reuse the same frequency at the adjacent cell. there is no need to prepare a separate frequency plan. Total interference generated on mobile station signals received from the cell site is the sum of interference generated from other mobile stations in the same cell site and interference generated from the mobile station of adjacent cell site. That is, each mobile station signal generates interference in relation to the signals of all the other mobile stations.

Total interference from all the adjacent cell sites is the ratio of interference from all the cell sites versus total interference from other mobile stations in the same cell site (about 65%). In the case of directional cell site, one cell normally uses a 120°sector antenna in order to divide the sector into three. In this case, each antenna is used only for 1/3 of mobile stations in the cell site and accordingly, interference is reduced by 1/3 on the average and the capacity that can be supported by the entire system is increased by three times.

## 2.7 Soft Capacity

The subscriber capacity of the CDMA system is flexible depending on the relation between the number of users and service classes. For example, the system operator can increase the number of channels available for use during the busy hour despite the drop in call quality. This type of function requires 40% of normal call channels in the standby mode during the handoff, in an effort to avoid call disconnection resulting from the lack of channels.

In the CDMA system, services and service charges are classified further into different classes so that more transmit power can be allocated to high class service users for easier call set-up; they can also be given higher priority of using hand-off function than the general users.

# 3. Structure and Functions of Six-mode CDMA/GSM Mobile Phone

The hardware structure of CDMA/GSM mobile phone is made up of radio frequency (RF) part and logic part. The RF part is composed of Receiver part (Rx), Transmitter part (Tx) and Local part (LO). For the purpose of operating on six-mode six-band, it is necessary five Tx path, six Rx path, dual PLL and switching system for band selection. The mobile phone antenna connects to the two switch modules that route signals according to the selected mode: DCS1800(PCS1900) Tx, EGSM Tx, DCS1800,PCS1900) Rx, EGSM Rx, CDMA, or GPS. The CDMA path includes a duplexer and two duplexers

First switch module(single pole three through) separates GSM, CDMA, and GPS modes. And second switch module(switch plexer) carries out separating EGSM Tx/Rx band and DCS1800(PCS1900) Tx/Rx band. Duplexer seperates CDMA DCN and CDMA USPCS.

The CDMA receive signals are amplified by the appropriate RFL6000 LNA then passes through a bandpass filter before applied to the RFR6000 Receiver IC. On-chip circuits downconvert the received signal directly from RF to baseband using direct conversion techniques. Generation of the CDMA downconverter LOs is distributed between the RTR6300(Phase-locked loop), the RFR6000(buffer amplifiers and LO distribution circuits), and external dual-band UHF VCO and loop filter circuits. The RFR6000 IC outputs analog baseband signals for further processing by the MSM device. This baseband interface is shared with the RTR6300 EGSM/DCS1800(PCS1900) receiver outputs.

The GPS LNA is integrated into the RFR6000 IC, so the GPS signal is routed directly from its antenna to the RFR through a bandpass filter. The RFR6000 IC also downconverts GPS signals directly to baseband, sharing the analog baseband output pins with CDMA received signals.

EGSM and DCS1800(PCS1900) receive signals pass through their bandpass filters then are applied to the RTR6300 IC. Similar to the CDMA receive paths, RTR6300 circuits downconvert the received signals directly from RF to baseband. The EGSM/DCS1800(PCS1900) downconverter LO is generated mostly within the RTR6300(PLL and distribution functions); the dual-band UHF VCO and loop filter are off-chip. The RTR analog baseband outputs are routed to the MSM6500 IC for further processing(an interface shared with the RFR)

The CDMA transmit paths begin with analog baseband signals from the MSM device that drive the RTR6300 IC. Integrated PLL and VCO circuits generate the Tx LO used in the quadrature upconverters that translate baseband signals directly to RF> The RTR6300 output driver stages deliver fairly high-level signals that are filtered and applied to the power amplifiers(PA). The PA outputs are protected by isolators and routed to the antenna through each band’s duplexer, a band splitting diplexer, and a switch module.

The shared EGSM-900 and DCS1800(PCS1900) transmit path begins with the same baseband interface from the MSM6500 IC that is used for the CDMA signals. A single EGSM/DCS1800 quadrature upconverter translates the GMSK-modulated signal to a convenient intermediate frequency(IF) that forms one input to an offset phase-locked loop(OPLL). OPLL functions are split between the RTR6300 IC and off-chip loop filter and dual Tx VCO circuits, and translate the GMSK-modulated signal to the desired EGSM-900 or DCS1800 channel frequency. This signal is applied to a dual power amplifier(only one is active at a time). The enabled path continues with the PA, an automated power control(APC) circuit samples the transmit power and adjusts its level, switch module(s), and the antenna.

Handset power supply voltages are managed and regulated by the PM6650 Power Management IC. This versatile device integrates all wireless handset power management, general housekeeping, and user interface support functions into a single mixed signal IC. It monitors and controls the external power source and coordinates battery recharging while maintaining the handset supply voltages using low dropout, programmable regulators.

The device’s general housekeeping functions include  an ADC and analog multiplexer circuit for  monitoring on-chip voltage sources, charging status, and current flow, as well as user-defined off-chip variables such as temperature, RF output power, and battery ID. Various oscillator, clock, and counter circuits support IC and higher-level handset functions. Key parameters such as under-voltage lockout and crystal oscillator signal presence are monitored to protect against detrimental conditions.

The Power Management IC includes backlight or LED drivers with brightness control that are typically used for keypad and LCD drivers. Independent vibrator and ringer drivers alert handset users of incoming calls, and a speaker driver with volume control supports speakerphone and melody ringer applications.

4. Specification

## 4.1 General Specification

### 4.1.1 Transmit/Receive Frequency Interval :

- CELLULAR : 45 MHz

- GSM : 45 MHz

- DCS1800 : 95 MHz

- PCS1900 : 60 MHz

### 4.1.2 Number of Channels (Channel Bandwidth)

- CELLULAR : 20 Channels

- GSM : 174 CH

- DCS1800 : 374 CH

- PCS1900 : 299CH

### 4.1.3 Operating Voltage : DC 3.3~4.2V

### 4.1.4 Battery Power Consumption (@ DC 3.7V)

|  | ### SLEEP | ### IDLE | ### MAX. POWER |
| --- | --- | --- | --- |
| ### CELLULAR | ### 1.5 mA | ### 130 mA | ### 600 mA (@ 24 dBm output) |
| ### US-PCS | ### 1.5 mA | ### 130 mA | ### 700 mA (@ 23 dBm output) |
|  | ### Standby mode |  | ### Talk Mode |
| ### GSM/DCS1800/PCS1900 | ### 5 mA ### (PP9) | ### 10 mA (PP2) | ### GSM PCL5 : 400 mA DCS1800 PCL0 : 320 mA |

### 4.1.5 Operating Temperature : -20C ~ +60C

### 4.1.6 Frequency Stability

- CDMA : ±0.5PPM

- GSM/DCS1800/DC1900 : ± 0.1PPM

### 4.1.7 Antenna : Stubby Type (Helical w/Meander, 50 )

### 4.1.8 Size and Weight

Size : 93.5×47.9×24.25mm

- Weight : 123g (Approximately with standard battery)

### 4.1.9 Channel Spacing

- CELLULAR : 1.25MHz

- GSM : 200 KHz

### 4.1.10 Battery Type, Capacity and Operating Time.

|  | ### Standard (1000 mAh) |  |
| --- | --- | --- |
| ### Standby Time | ### CELLULAR | ### About 140 Hours (@ SCI=1) |
|  | ### GSM/DCS1800/1900 | ### About 200 Hours (@PP=9) |
| ### Talk Time | ### CELLULAR | ### 170 Minutes (@ -92 dBm,half rate) |
|  | ### GSM | ### 150 Minutes (@PCL 5) |
|  | ### DCS1800/1900 | ### 180 Minutes (@PCL 0) |

## 4.2 Receive Specification

### 4.2.1 Frequency Range

- CELLULAR : 869.820 MHz ~ 893.190 MHz

- US-PCS(CDMA) : 1930MHz ~ 1990MHz

- GSM : 925.2 MHz ~ 959.8 MHz

- DCS1800 : 1805.2 MHz ~ 1879.8 MHz

- PCS1900 : 1850 ~ 1990 MHz

- GPS : 1575.42 MHz

### 4.2.2 Local Oscillating Frequency Range :

- CELLULAR : 1738.08 MHz ~ 1787.94 MHz

- US-PCS(CDMA) : 1715MHz ~ 1786.94MHz

- GSM : 0 Hz

- DCS1800 : 0 MHz

- PCS1900 : 0 MHz

- GPS : 3150.84 MHz

### 4.2.3 Sensitivity

- CELLULAR : -104 dBm (C/N 12dB or more)

- US-PCS(CDMA) : -104 dBm (C/N 12dB or more)

- GSM : -102 dBm

- DCS1800 : -102 dBm

- PCS1900 : -102 dBm

- GPS : -150 dBm (w/o SA), -154 dBm (w/SA)

### 4.2.4 Selectivity

- CELLULAR : 3dB C/N Degration (With Fch±1.25 kHz : -30dBm)

### 4.2.5 Interference Rejection

- Single Tone : -30dBm at 900 kHz (CELLULAR)

IMD : -43dBm at 900 kHz & 1700kHz(CELLULAR),

-32dBm at at 900 kHz & 1700kHz, -21dBm at 900 kHz & 1700kHz

### 4.2.6 Spurious Wave Suppression : Maximum of -80dB

### 4.2.7 CDMA Input Signal Range

- Dynamic area of more than -104~ -25 dB: 79dB at the 1.23MHz band.

- Transmit Band : below –60dBm

- Receive Band : below –80dBm

### 4.2.8 GSM Rx Level Report Accuracy

- ±4 dB

### 4.2.8 GSM Rx Level Report Accuracy

- GSM : Dynamic area of more than –102 ~-15 dB : 87 dB at the 200 KHz band.

- DCS1800 : Dynamic area of more than –100 ~ -23 dB : 77 dB at 200 KHz band.

- PCS1900 : Dynamic area of more than –100 ~ -23 dB : 77 dB at 200 KHz band

### 4.3 Transmit Specification

### 4.3.1 Frequency Range

- CELLULAR : 824.820MHz ~ 848.190MHz

- US-PCS(CDMA) : 1850MHz ~ 1910MHz

- GSM : 880.2 MHz ~ 914.8 MHz

- DCS1800 : 1710.2 MHz ~ 1784.8 MHz

- PCS1900 : 1850.2 MHz ~ 1908.8 MHz

- GPS : 1575.42 MHz

### 4.3.2 Output Power

- CELLULAR : 0.224 W

- US-PCS(CDMA) : 0.224W

- GSM : 2 W

- DCS1800 : 1W

- PCS1900 : 1W

### 4.3.3 CDMA TX Frequency Deviation :

1) CELLULAR : +300Hz or less

### 4.3.4 CDMA TX Conducted Spurious Emissions

- CELLULAR : 900kHz : -42 dBc/30kHz below

1.98MHz : -54 dBc/30kHz below

### 4.3.5 CDMA Minimum Tx Power Control

### 1) CELLULAR : -50dBm below

### 4.3.6 GSM,DCS1800,PCS1900 Conducted Spurious Emissions

| ### Frequency Range | ### Power Level in dBm |  |  |
| --- | --- | --- | --- |
|  | ### GSM900 | ### DCS1800 | ### PCS1800 |
| ### 100KHz to 1GHz | ### -36 | ### -36 | ### -36 |
| ### 1GHz to 12.75GHz | ### -30 |  |  |
| ### 1GHz to 1710MHz |  | ### -30 | ### -30 |
| ### 1710MHz to 1785MHz |  | ### -36 | ### -30 |
| ### 1785MHz to 1850MHz |  | ### -30 | ### -30 |
| ### 1850MHz to 1910MHz |  |  | ### -36 |
| ### 1910MHz to 12.75GHz |  |  | ### -30 |

Unit : dBm

### 4.3.7 GSM/DCS1800/PCS1900 Power Level

| GSM Band |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Level | Power | Toler. | Level | Power | Toler. |
| 5 | 33dBm | ±2dB | 13 | 17dBm | ±3dB |
| 6 | 31dBm | ±3dB | 14 | 15dBm | ±3dB |
| 7 | 29dBm | ±3dB | 15 | 13dBm | ±3dB |
| 8 | 27dBm | ±3dB | 16 | 11dBm | ±5dB |
| 9 | 25dBm | ±3dB | 17 | 9dBm | ±5dB |
| 10 | 23dBm | ±3dB | 18 | 7dBm | ±5dB |
| 11 | 21dBm | ±3dB | 19 | 5dBm | ±5dB |
| 12 | 19dBm | ±3dB |  |  |  |

| DCS1800(PCS1900) Band |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Level | Power | Toler. | Level | Power | Toler. |
| 0 | 30dBm | ±2dB | 8 | 14dBm | ±3dB |
| 1 | 28dBm | ±3dB | 9 | 12dBm | ±4dB |
| 2 | 26dBm | ±3dB | 10 | 10dBm | ±4dB |
| 3 | 24dBm | ±3dB | 11 | 8dBm | ±4dB |
| 4 | 22dBm | ±3dB | 12 | 6dBm | ±4dB |
| 5 | 20dBm | ±3dB | 13 | 4dBm | ±4dB |
| 6 | 18dBm | ±3dB | 14 | 2dBm | ±5dB |
| 7 | 16dBm | ±3dB | 15 | 0dBm | ±5dB |

4.3.8 Output RF Spectrum Dut to Modulation

| EGSM |  | DCS1800 |  |
| --- | --- | --- | --- |
| Offset from Carrier (KHz) | Max.dBc | Offset from Carrier (KHz) | Max.dBc |
| 100 | +0.5 | 100 | +0.5 |
| 200 | -30 | 200 | -30 |
| 250 | -33 | 250 | -33 |
| 400 | -60 | 400 | -60 |
| 600 ~ 1200 | -60 | 600 ~ 1200 | -60 |
| 1200 ~ 1800 | -60 | 1200 ~ 1800 | -60 |
| 1800 ~ 3000 | -63 | 1800 ~ 3000 | -65 |
| 3000 ~ 6000 | -65 | 3000 ~ 6000 | -65 |
| 6000 | -71 | 6000 | -73 |

4.3.9 Output RF Spectrum Dut to Switching Transient

| EGSM |  | DCS1800(PCS1900) |  |
| --- | --- | --- | --- |
| Offset from Carrier (KHz) | Max.dBm | Offset from Carrier (KHz) | Max.dBm |
| 400 | -19 | 400 | -22 |
| 600 | -21 | 600 | -24 |
| 1200 | -21 | 1200 | -24 |
| 1800 | -24 | 1800 | -27 |

## 4.4 Acoustic Specification

4.4.1 SLR(Sending Loudness Rating) / RLR(Receiving Loudness Rating)

8  3 dB / 2  3 dB

4.4.2 Sending Response and Receiving Response

| Sending Response |  |  | Receiving Response |  |  |
| --- | --- | --- | --- | --- | --- |
| Frequency (Hz) | Max.(dB) | Min(dB) | Frequency (Hz) | Max.(dB) | Min(dB) |
| 100 | -12 | / | 100 | -12 | / |
| 200 | 0 | / | 200 | 0 | / |
| 300 | 0 | -12 | 300 | 2 | -7 |
| 1000 | 0 | -6 | 1000 | * | -5 |
| 2000 | 4 | -6 | 2000 | 0 | -5 |
| 3000 | 4 | -6 | 3000 | 2 | -5 |
| 3400 | 4 | -9 | 3400 | 2 | -10 |
| 4000 | 0 | / | 4000 | 2 |  |

* mean that Adopt a straight line in between 300Hz and 1000 Hz to be Max. level in the range.

4.4.3 STMR(SideTone Masking Rating)

13  5 dB

4.4.4 Stability Margin

＞6dB

4.4.5 Distortion

| dB to ARL (dB) | Level Ratio (dB) |
| --- | --- |
| -35 | 17.5 |
| -30 | 22.5 |
| -20 | 30.7 |
| -10 | 33.3 |
| 0 | 33.7 |
| 7 | 31.7 |
| 10 | 25.5 |

4.4.6 Side tone Distortion

Three stage distortion ＜ 10%

## 4.5 MS (Mobile Station) Transmitter Frequency

GSM : 890 + (n - 1024)  0.2 MHz

DCS1800 : 1710 + (n-512)  0.2 MHz

PCS1900 :  1850 + (n-512)  0.2 MHz

## 4.6 MS (Mobile Station) Receiver Frequency

GSM : 935 + (n - 1024)  0.2 MHz (n = 975  1024)

DCS1800 : 1805 + (n-512)  0.2 MHz (n = 512  885)

PCS1900 :  1930 + (n-512)  0.2 MHz (n = 512  810)

## 4.7 MS (Mobile Station) Transmitter Frequency

## 4.7.1 CELLULAR mode

| Ch # | Center Freq. (MHz) | Ch # | Center Freq. (MHz) |
| --- | --- | --- | --- |
| 1011 29 70 111 152 193 234 275 316 363 | 824.640 825.870 827.100 828.330 829.560 830.790 832.020 833.250 834.480 835.890 | 404 445 486 527 568 609 650 697 738 779 | 837.120 838.350 839.580 840.810 842.040 843.270 844.500 845.910 847.140 848.370 |

## 4.8 MS (Mobile Station) Receiver Frequency

## 4.8.1 CELLULAR mode

| Ch. # | Center Freq. (MHz) | Ch. # | Center Freq. (MHz) |
| --- | --- | --- | --- |
| 1011 29 70 111 152 193 234 275 316 363 | 869.640 870.870 872.100 873.330 874.560 875.790 877.020 878.250 879.480 880.890 | 404 445 486 527 568 609 650 697 738 779 | 882.120 883.350 884.580 885.810 887.040 888.270 889.500 890.910 892.140 893.370 |

4.8.3 GPS mode : 1575.42MHz

# CHAPTER 2. Circuit Description

# 1. RF Transmit/Receive Part

## 1.1 Overview

LG-UW810 is roaming hand set with six-mode, CDMA DCN, CDMA USPCS, GPS, EGSM, DCS1800 and PCS1900.  The Tx and Rx part employ the Direct-Conversion system. First of all, the CDMA Tx and Rx frequencies are respectively 824.04~848.97 and 869.04~893.97 for Cellular. RF signals received through the antenna are seperated by the SP3T switch. CDMA RF Signal fed into the low noise amplifier (LNA) through the duplexer. Then, they are fed into RFR6000. In RFR6000, the base band signal is changed into baseband signal directly. Then, this signal is changed into digital signal by the analog to digital converter (ADC, A/D Converter), and the digital circuit part of the MSM(Mobile Station Modem) 6500 processes the data from ADC. The digital processing part is a demodulator.

Moreover, EGSM/DCS1800(PCS1900) RF signals are passed along GSM switchplexer and then fed into RTR6300 with differential inputs across the each BPF (Band Pass Filter). In RTR(RF Transmiter Receiver)6300 chip set, Both paths begin with gain-stepped LNA circuit. Gain control is provided through software and the sarial interface. The LNA outputs drive the RF ports of quadrature RF-to-baseband downconverters. The downconverted baseband outputs are multiplexed and routed to lowpass filters (one I and ond Q)whose passband stopband chracteristics supplement MSM processing. In the case of transmission, RTR6300 receives OQPSK-modulated anlaog signal from the MSM6500.  These analog baseband signals are amplifed, filtered, and then multilexed to the Cellular upconvertor mixers. After that, the RF signal is amplified by the Power Amplifier in order to have enough power for radiation. Finally, the RF signal is sent out to the cell site via the antenna after going through the duplexer. Meanwhile, EGSM-900 and DCS1800-1800 transmit path begins with the same base band interface from the MSM6500 IC that used for cdma2000 signals. The differential analog input signals are buffered, filtered by a set of lowpass filters, corrected for DC offsets then appied to the EGSM/DCS1800 quadrature upconverter. The upconverter translates the GMSK-modulated signal to a convenient intermidiate frequency (IF) that forms one input to a frequency/phase detector circuit. This IF signal is the reference input to an offset phase-locked loop (OPLL) circuit. This locked RF signals are amplifed at Power Amplifier and tranmit the air interface across the switchplexer, SP3T and the antenna.

## 1.2 Description of Rx Part Circuit

### 1.2.1 SP3T(Single Pole Triple Throw) RF switch (U1101)

The main function of SP3T switch is to prohibit the other band signals from flowing into the one band circuit and vice versa. RF designer can use common tri-band antenna regardless of frequency band (800, 1575 and 1900 MHz). The specification of LG-UW810 SP3T switch is described below :

|  | Cellular | GPS | GSM/DCS1800/PCS1900 |
| --- | --- | --- | --- |
| Frequency Range | 824 – 894 MHz | 1575.42 MHz | 880 – 960MHz 1850 – 1990 MHz |
| Insertion Loss to Common | 0.58 dB (@ 25°C) | 0.64 dB (@ 25°C) | 0.68 dB (@ 25°C) |
| Isolation | -28 dB | -23 dB | -22 dB |
| Return Loss | -25 dB |  |  |
| Power Capacity | 38 dBm Max. |  |  |
| Temperature Range | -45°C to +85°C |  |  |

### 1.2.2 Duplexer / GSM switchplexer (DP1102,DP1103,U1118)

The duplexer consists of the Rx bandpass filter (BPF) and the Tx BPF which has the function of separating Tx and Rx signals in the full duplex system for using the common antenna. The Tx part BPF is used to suppress noises and spurious out of the Tx frequency band. The Rx BPF is used to receive only Rx signal coming from the antenna, which is usually called preselector. It’s main function is to limit the bandwidth of spectrum reaching the LNA and mixer, attenuate receiver spurious response and suppress local oscillator energy. As a result frequency sensitivity and selectivity of mobile phone increase. The specification of LG-UW810 duplexer  described below :

- CDMA duplexer

|  | Tx | Rx | Tx to Rx (min.) |
| --- | --- | --- | --- |
| #### Pass Band | 824~849 MHz | 869~894 MHz |  |
| Insertion Loss | 2.3 dB max. | 3.0 dB max. |  |
| Return Loss | 12.0 dB min. | 10.0 dB min. |  |
| Attenuation | 42 dB min . (849~894MHz) | 52 dB min. (824~849MHz) | 56dB (824~849MHz) 40 dB (1930~1990MHz) |

The GSM switchplexer is made up of one diplexer, two lowpass filters and RF-active switches. The diplexer separates to low frequency and high frequency. For reason, GSM wireless communication is available for both TDMA (Time Division Multiple Access) and FDMA (Frequency Division Multiple Access). RF-active switches play a part that separates Tx and Rx for both low frequency and high frequency. Finally, Lowpass filters are suppressed 2nd and 3rd harmonics for both low frequency and high frequency, which resulted in fundamental frequency. For EGSM output power class is 4 (+33 dBm), the power capacity of switchplexer is important parameter.

- GSM ASM(antenna switch module)

<EGSM band>

|  | Tx | Rx | Tx to Rx (min) |
| --- | --- | --- | --- |
| #### Pass Band | 880~915MHz | 925~960 MHz |  |
| Insertion Loss | 1.2 dB max | 1.0 dB max |  |
| VSWR | 2.0 max | 2.0 max |  |
| Attenuation | 30 dB min (2nd, 3rd MHZ) |  | 20.0 min (Tx-Ant) |
| Power capacity | 35.0 max |  |  |

<DCS1800 band>

|  | Tx | Rx | Tx to Rx (min) |
| --- | --- | --- | --- |
| #### Pass Band | 1710~1785MHz | 1805~1880 MHz |  |
| Insertion Loss | 1.2 dB max | 1.0 dB max |  |
| VSWR | 2.0 max | 2.0 max |  |
| Attenuation | 30 dB min (2nd, 3rd MHZ) |  | 20.0 min (Tx-Ant) |
| Power capacity | 33.0 max |  |  |

<PCS1900 band>

|  | Tx | Rx | Tx to Rx (min) |
| --- | --- | --- | --- |
| #### Pass Band | 1850~1910 MHz | 1930~1990MHz |  |
| Insertion Loss | 1.2 dB max | 1.0 dB max |  |
| VSWR | 2.0 max | 2.0 max |  |
| Attenuation | 30 dB min (2nd, 3rd MHZ) |  | 20.0 min (Tx-Ant) |
| Power capacity | 33.0 max |  |  |

### 1.2.3 LNA (U1105)

The RFL6000 has Cellular and PCS LNAs, respectively.  The characteristics of Low Noise Amplifier (LNA) are low noise figure, high gain, high intercept point and high reverse isolation. The frequency selectivity characteristic of mobile phone is mostly determined by LNA.  The specification of LG-UW810 LNAs are described below ;

| Parameter | Low gain |  | Middle gain |  | High gain |  | Units |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Cellular | PCS | Cellular | PCS | Cellular | PCS |  |
| Gain | -18.45 | -19.78 | -1.57 | -8.69 | 15.44 | 16.23 | dB |
| Noise Figure | 18.91 | 20.68 | 2.21 | 9.63 | 1.44 | 1.41 | dB |
| Input IP3 | 25.73 | 32.66 | 23.98 | 25.23 | 11.61 | 6.34 | dBm |

1.2.4 RFR6000 (U1115)and GPS LNA (U1106)

The RFR6000 device performs signal down-conversion for Cellular, PCS and GPS tri-band applications.  It contains all the circuitry (with the exception of external filters) needed to support conversion of received RF signals to baseband signals. The three down-converted Mixers (Cellular, PCS and GPS), and an LO Buffer Amplifier to buffer the RF VCO to the RF Transmit Up-converter. The GPS LNA & mixers offer the most advanced and integrated CDMA Rx solution designed to meet cascaded Noise Figure (NF) and Third-order Intercept Point (IIP3) requirements of IS-98E for Sensitivity, Two-Tone Inter-modulation, and Single-tone Desensitization.

### 1.2.5 Rx RF SAW FILTER(F1103, F1105, F1100, F1101)

The main function of Rx RF SAW filter is the bandpass filter to attenuate mobile phone spurious frequencies and harmonics, attenuate noise amplified by the LNA and suppress second harmonic originating in the LNA.

## 1.3 Description of Transmit Part Circuit

### 1.3.1 RTR6300 (U1116)

The RTR6300 is an RF transceiver IC, baseband-to-RF Transmit Processor which supports multi-band, multi-mode phone with two receiver signal paths and four transmitter signal paths. : Receiver paths are EGSM-900 and DCS1800. Also, four transmitter paths are CDMA Cellular/ PCS band, EGSM/ DCS1800 band. The EGSM and DCS1800 receiver inputs are connected directly to the transceiver front-end circuits (filters and switch module). On the transmit side, the EGSM and DCS1800 transmitter paths share a two-pin interface that supplements off-chip VCO (Tx-VCO) and power divider circuits to implement an offset-PLL (OPLL) off-chip components : a dual-band power amplifier, coupler, automatic power control circuits, and the antenna switch module. Transceiver LO signals are generated and distributed to the quadrature up and down converters using one of two VCO outputs. The on-chip VCO is controlled by the first phase-locked loop (PLL1) and supports cdma2001x modes. An off-chip UHF VCO is controlled by PLL2 to support EGSM Rx and Tx, DCS1800 Rx and Tx, and cdma2000 Rx modes. PLL2 also supports Gpsone operation by controlling the RFR6000 on-chip VCO. Both PLL circuits are integrated into the RTR6300, satisfying all handset PLL requirements without a separate PLL IC. The loop filter circuits are implemented off-chip to allow customization for specific handset applications. In case of cdma2000, the baseband quadrature signals are upconverted to the Cellular or PCS frequency bands and amplified to provide signal drive capability to the PAM. The RTR6300 includes an mixers for up-converting analog baseband to RF, a programmable PLL for generating Tx and Rx LO frequency, Cellular and PCS driver amplifiers and Tx power control through an 85 dB VGA. As added benefit, the single sideband upconversion eliminates the need for a band pass filter normally required between the upconverter and driver amplifier.

- RTR6300 Cellular and PCS CDMA RF Specifications

| Parame | Condition | Min. | Typ. | Max. | Units |
| --- | --- | --- | --- | --- | --- |
| Max Output Power (CDMA) | Average CDMA Cellular Average CDMA PCS | 8 10 |  |  | dBm dBm |
| Input Power (GSM) | Average EGSM Average DCS1800 |  | -15 -25 |  | dBm dBm |
| Min Output Power | Average CDMA Cellular Average CDMA PCS Average EGSM Average DCS1800 |  | -18 -18 | -75 -75 | dBm dBm dBm dBm |
| Rx band noise power (CDMA) | CDMA Cellular CDMA PCS |  | -134 -132 |  | dBm /Hz |
| Open loop phase noise (GSM) | Average EGSM(10MHz offset) Average EGSM(20MHz offset) Average DCS1800(20MHz offset) |  | -133 -136 -133 |  | dBm /Hz |
| ACPR | Cellular: Fc885kHz PCS: Fc1.25MHz |  | -53 -50 |  | dBc/ 30kHz |

### 1.3.2 Power Amplifier(U1107,U1108, U11104)

The power amplifier that can be used in the EGSM/DCS1800/PCS1900 and CDMA mode has linear amplification capability and high efficiency. For higher efficiency, it is made up of one MMIC (Monolithic Microwave Integrated Circuit) for which RF input terminal and internal interface circuit are integrated onto one IC after going through the GaAs HBT (heterojunction bipolar transistor) process. The module of power amplifier is made up of an output end interface circuit including this MMIC. The maximum output power is +35Bm/+33dBm in EGSM/DCS1800/PCS1900 and +28dBm in CDMA. CDMA RF transmit signals that have been amplified through the power amplifier are sent to the duplexer and EGSM/DCS1800/PCS1900 to the GSM switchplexer.

## 1.4 Description of Frequency Synthesizer Circuit

### 1.4.1 Voltage Control Temperature Compensation Crystal Oscillator (VCTCXO, U1114)

The temperature variation of mobile phone can be compensated by VCTCXO. The reference frequency of a mobile phone is 19.2 MHz.  The receiver frequency tuning signals called TRK_LO_ADJ from MSM as 0.5 V~2.5 V DC via R and C filter in order to generate the reference frequency of 19.2 MHz and input it into the frequency synthesizer.  Frequency stability depending on temperature is ±2.0 ppm.

### 1.4.2 Voltage Controlled Oscillator (UHF dual VCO, U1113)

The external VCO signal is processed by the LO generation and distribution circuits in RFR6000 to create the EGSM/DCS1800 Rx and Cellular Rx quadrature downconverter’s LO signals.  Likewise, the internal VCO signal of RFR6000 is processed to create the GPS quadrature downconverter’s LO signal.  In all cases, the LO signals applied at the mixer ports are at the frequency different than the VCO frequency.  This assures that the VCO frequency is different than the RF frequency, an important consideration for Zero-IF processing.  The VCO frequency used are 1635~1788 MHz for GSM/DCS1800 and Cellular and they are produced in single voltage controlled oscillator of U1113.

### 1.4.3 Dual Tx Voltage Controlled Oscillator (Dual Tx VCO, U1109)

The waveform at the dual Tx VCO signal is the GMSK-modulated signal centered at the desired EGSM-900 or DCS1800/PCS1900 channel frequency. A phase=locked loop circuit is used to translate the GMSK- modulated signal from IF to RF primarily for two reasons ;  one is that phase-locked loops provide a lowpass filter function from the reference input to the VCO output. This results in a bandpass function centered at the desired channel frequency that provides steep, well-controlled rejection of the out-of-band spectrum. The other that the resulting output bandpass function is virtually unchanged as the transmitter is tuned over channels spannig the EGEM or DCS1800/PCS1900 operating band. These signals are applied to both power amplifiers, though only one is active at a time. The enabled path continues with the PA, the switchplexer. (which includes a band-appropriate lowpass filter and switches), and antenna.

# 2. Digital/Voice Processing Part

## 2.1 Overview

The digital/voice processing part processes the user's commands and processes all the digital and voice signal processing in order to operate in the phone. The digital/voice processing part is made up of a keypad/LCD, receptacle part, voice processing part, mobile station modem part, memory part, and power supply part.

## 2.2 Configuration

### 2.2.1 Keypad/LCD and Receptacle Part

This is used to transmit keypad signals to MSM6500. It is made up of a keypad backlight part that illuminates the keypad, LCD part that displays the operation status on to the screen, and a receptacle that receives and sends out voice and data with external sources.

### 2.2.2 Voice Processing Part

The voice processing part is made up of an audio codec in MSM6500 used to convert MIC signals into digital voice signals and digital voice signals into analog voice signals,

amplifying parts for amplifying the voice signals and MIC signals are on Codec in MSM6500.

### 2.2.3 MSM6500 (Mobile Station Modem) Part

MSM6500 is the core element of a CDMA/GSM mobile station and carries out the functions of CPU, encoder, interleaver, deinterleaver, Viterbi decoder, Mod/Demod, codec, and vocoder.

### 2.2.4 Memory Part

- The memory part is made up of a NAND flash memory and a SDRAM

### 2.2.5 Power Supply Part

The power supply part is made up of a CMOS regulator, a DC/DC converter, two charge pumps and a PMIC.

The CMOS regulator gives the power to the Camera part.

The DC/DC converter gives the power to the MSM and memory parts.

The charge pump 1 gives the power to the Camera Flash part.

The charge pump 2 gives the power to the LCD back light LED and the Indicated LEDs.

The PMIC(PM6650) is made up of 8 Regulators and direct connet to Batt.

Regulator 1(50mA) gives the power to the MSM and memory parts.

Regulator 3(150mA ) gives the power to the MSM audio part.

Regulator 4(75mA) gives the power to the LCD and the UIM parts.

Regulator 5(35mA) gives the power to the TCXO part.

Regulator 6(30mA) gives the power to the PLL part of RTR6300.

Regulator 7(175mA) gives the power to the TX part.

Regulator 8(75mA) gives the power to the RX part.

PAM, Motor, keypad LED and Audio amplifier are directly conneted to Battery.

## 2.3 Circuit Description

### 2.3.1 Keypad/LCD and Receptacle Part

Once the keypad is pressed, the key signals are sent out to MSM6500 for processing. In addition, when the key is pressed, the keypad lights up through the use of 14 LEDs. The status and operation of a mobile station are displayed on the screen for the user with the characters and icons on the LCD.

Receptacle(CON757) exchanges audio signals and data with external sources and then, receives power from the battery or external batteries.

- 2.3.2 MSM Part

MSM6500 is the core element of a CDMA/GSM mobile station. Subsystems within the MSM6500 include a CDMA/GSM processor,a QCELP(qualcomm Code Excited Linear Predication) vocoder,an EVRC(Enhanced Variable Rate Codec) vocoder, an ARM9TDMI microprocessor ,and assorted peripheral interfaces that are used to support other functions.

The CPU controls total operations of the subscriber unit. Digital voice data, that have been inputted, are encoded using the QCELP algorithm. Then, they are convolutionally encoded so that error detection and correction are possible. Coded symbols are interleaved in order to avoid a burst error. Each data channel is scrambled by the long code PN sequence of the user in order to ensure the confidentiality of calls.

Moreover, binary quadrature codes are used based on Walsh functions in order to discern each channel. Data created thus are 4-phase modulated by one pair of Pilot PN code and they are used to create I and Q data.

When received, I and Q data are demodulated into symbols by the demodulator and then, de-interleaved in reverse to the case of transmission. Then, the errors of data received from Viterbi decoder are detected and corrected. They are voice decoded at the vocoder in order to output digital voice data.

The MSM6500 also supports Enhanced Variable Rate Coder (EVRC) operation in addition to the standard 8k and 13k vocoding rating.

### 2.3.2.1 Audio Processing Part

MIC signals are inputted into the audio codec, and amplified with programmable gain, and converted into digital signals(PCM). Then, they are inputted into MSM6500.

In addition, digital audio signals(PCM) outputted from MSM6500 are converted into analog signals after going through the audio codec. These signals are amplified with programmable gain on codec’s internal AMP and external Audio AMP and then transferred to the ear piece. The signals is generated in MSM6500 using CMX.

### 2.3.4 Memory Part

The memory part consists of a Combo Memory (1024Mbits NAND Flash Memory + 512Mbits SDRAM). On the NAND Flash Memory of the MCP Memory, there are programs used for the operation of a mobile station and the non-volatile data of the mobile station such as a ESN(Electronic Serial Number) are stored. The programs can be changed through down loading after the assembling of mobile stations. On the SDRAM, data generated during the operation of a mobile station are stored temporarily.

### 2.3.5 Power Supply Part

Turn ON

When the battery voltage (4.20V ~ 3.32V) is fed and the PWR key of keypad is pressed, PMIC is activated by the ON_SW signal, and then the control signal ON_SW_S/ signal is generated. And then, the regurator +1.4V_MSMC & +1.8V_MSMP1 & +2.6V_MSMP2, +2.6V_MSMA, are operated.

Operating

During the phone is on operating state,

Two LDOs(in PMIC) and two DC/DC converter for MSM are always enable and gives the power MSM6500 and memory part.

LDO(in PMIC) for +2.85V_TX part is enabled on IDLE state, and gives the power TX part devices.

LDO(in PMIC) for +2.85V_RX part is enabled on SLEEP state, and gives the power RX part devices.

Turn OFF

When the PWR key is pressed during a few seconds, PMIC sends the signal of Turn Off. MSM6500 receives this signal and then, recognizes that the POWER key has been pressed. During this time, MSM6500 outputs PS_HOLD as low and turn off all devices

### 2.3.6 Logic Part

The Logic part consists of internal CPU of MSM6500, MCP(SDRAM & NAND FLASH MEMORY) .

The MSM6500 receives TCXO/4 clock(19.20Mz) and CHIPX8 clock signals, and then controls the phone during the CDMA and the GSM mode. The major components are as follows:

CPU : ARM9TDMI microprocessor core

MEMORY :  1024Mbits NAND + 512Mbits SDRAM (TY9000A410AMGF)

- CPU

ARM9TDMI 32-bit microprocessor is used and CPU controls all the circuitry. Some of the features of the ARM microprocessor include a 3 stage pipelined RISC architecture, both 32-bit ARM and 16bit THUMB instruction setsm, a 32-bit address bus, and a 32-bit internal data bus.

- NAND FLASH Memory

Flash Memory is used to store the program of the mobile station. Using the down-loading program, the program can be changed even after the mobile station is fully assembled.

- SDRAM

- SDRAM is used to store the internal flag information, call processing data, and timer data.

- KEYPAD

For key recognition, key matrix is setup using KEY_SENSE0-4_N signals and GPIOs of output ports of MSM. Backlight circuitry are included in the keypad for easy operation in the dark.

- LCD MODULE

LCD module contains a controller which will display the information onto the LCD by 16-bit data from the MSM.

It has TFT LCD 260K colors (main 320 X240 dots , sub 128 X 128 dots).

- CAMERA MODULE

Camera module has Maicovicon sensor(progresive scanning sensor) which has 130M pixels. It sends image data to MSM via the dedicated camera interface.

- UIM / SIM Part

UIM part is composed of 2 UIM/SIM card sockets.

UIM/SIM cards contain the informations of phone number, PIM data, SMS data, etc.

# CHAPTER 3. Trouble Shooting

※ Refer to “LG-UW810 Trouble Shouting.exe” file

# CHAPTER 4. Glossary

## General Terms

Abbreviated Alert 	An abbreviated alert is used to remind the mobile station user that previously selected alternative routing features are still active.

AC 			See Authentication Center.

Access Attempt   	A sequence of one or more access probe sequences on the Access Channel containing the same message. See also Access Probe and Access Probe Sequence.

Access Channel 	A Reserve CDMA Channel used by mobile stations for communicating	to the base station. The Access Channel is used for short signaling    message exchanges such as call origination’s, responses to pages, and registrations. The Access Channel is a slotted random access channel.

Access Channel Message 	The information part of an access probe consisting of the message body, length field, and CRC.

Access Channel Message Capsule 	An Access Channel message plus the padding.

Access Channel Preamble 	The preamble of an access probe consisting of a sequence of all-zero frames that is sent at the 4800 bps rate.

Access Channel Request Message  An  Access Channel message that is	generated by the mobile station. See also Access Channel Response Message.

Access Channel Response Message 	A message on the Access Channel generated to reply to a message received from the base station.

Access Channel Slot 	The assigned time interval for an access probe. An Access Channel slot consists of an integer number of frames. The transmission of an access probe is performed within the boundaries of an Access Channel slot.

Access Probe        One Access Channel transmission consisting of a preamble and a message. The transmission is an integer number of frames in length and transmits one Access Channel message. See also Access Probe Sequence and Access Attempt.

Access Probe Sequence 	A sequence of one or more access probes on the Access Channel. The same Access Channel message is transmitted in every access probe of an access attempt. See also Access Probe and Access Attempt.

Acknowledgement 	A Layer 2 response by the mobile station or the base station confirming            that a signaling message was received correctly.

Action Time 		The time at which the action implied by a message should take effect.

Active Set  	The set of pilots associated with the CDMA Channels containing Forward Traffic Channels assigned to a particular mobile station.

ADC			      Analog to digital converter

Aging 	A mechanism through which the mobile station maintains in its Neighbor Set the pilots that have been recently sent to it from the base station and the pilots whose handoff drop timers have recently expired.

A-key 	A secret, 64-bit pattern stored in the mobile station. It is used to generate update the mobile station’s Shared Secret Data. The A-key is used in the mobile station authentication process.

AMPS			      Advanced Mobile Phone Service

Analog Access Channel 	An analog control channel used by a mobile station to access a system to obtain service.

Analog Color-Code 	An analog signal (see Supervisory Audio Tone) transmitted by a base station on an analog voice channel and used to detect capture of a mobile station by an interfering base station or the capture of a base station by an interfering mobile station.

Analog Control Channel 	An analog channel used for the transmission of digital control information from a base station to a mobile station or from a mobile station to a base station.

Analog Paging Channel 	A forward analog control channel that is used to page mobile stations and send orders.

Analog Voice Channel 	An analog channel on which a voice conversation occurs and on which brief digital messages may be sent from a base station to a mobile station or from a mobile station to a base station.

ANSI	     	            American National Standard Institute

Authentication 	A procedure used by a base station to validate a mobile station’s identity.

Authentication Center (AC)  An entity that manages the authentication information related to the mobile station.

Authentication Response (AUTHR) 	An 18-bit output of the authentication algorithm.	It is used, for example, to validate mobile station registrations, origination and terminations. A method of registration in which the mobile station registers without an explicit command from the base station.

AWGN 		Additive White Gaussian Noise.

Bad Frames 	Frames classified as erasures (frame category 10) or 9600 bps frames, primary traffic only with bit errors (frame category 9). See also Good Frames.

Base Station 	A station in the Domestic Public Cellular Radio Telecommunications Service, other than a mobile station, used for communicating with mobile stations. Depending upon the context, the term base station may refer to a cell, a sector within a cell, an MSC, or other part of the cellular system. See also MSC.

Base Station Authentication Response (AUTHBS) 	An 18-bit pattern generated by the authentication algorithm. AUTHBS is used to confirm the validity of base station orders to update the Shared Secret Data.

Base Station Random Variable (RANDBS) 	A 32-bit random number generated by the mobile station for authenticating base station orders to update the Shared Secret Data.

BCH Code 		See Bose-Chaudhuri-Hocquenghem Code.

BSC			Base Station Controller

BTS			Base Station Transmission System

Busy-Idle Bits 	The portion of the data stream transmitted by a base station on a forward analog control channel that is used to indicate the current busy-idle status of the corresponding reverse analog control channel.

Call Disconnect 	The process that releases the resources handling a particular call. The disconnect process beings either when the mobile station user indicates the end of the call by generating an on-hook condition or other call release mechanism, or when the base station initiates a release.

Call History Parameter (COUNT) 	A modulo-64 event counter maintained by the mobile station and Authentication Center that is used for clone detection.

Candidate Set 	The set of pilots that have been received with sufficient strength by the mobile station to be successfully demodulated, but have not been placed in the Active Set by the base station. See also Active Set. Neighbor Set, and Remaining Set. See Code Division Multiple Access

CDMA		Code Division Multiple Access

CDMA Channel 	The set of channels transmitted between the base station within	a given CDMA frequency assignment. See also Forward CDMA Channel and Reverse CDMA Channel.

CDMA Channel Number 	An 11-bit number corresponding to the center of the CDMA frequency assignment.

CDMA Frequency Assignment  	A 1.23MHz segment of spectrum centered on one of the 30KHz channels of the existing analog system.

Code Channel 	A subchannel of a Forward CDMA Channels. A Forward CDMA Channel contains 64 code channels. Code channel zero is assigned to the Pilot Channel. Code channels 1 through 7 may be assigned to the either Paging Channels or the Traffic Channels. Code Channel 32 may be assigned to either a Sync Channel or a Traffic Channel. The remaining code channels may be assigned to Traffic Channels.

Code Division Multiple Access (CDMA)  	A technique for spread-spectrum multiple-access digital communications that creates channels through the use of unique code sequences.

Code Symbol  	The output of an error-correcting encoder. Information bits are input to the encoder and code symbols are output from the encoder. See Convolutional Code.

Continuous Transmission   A mode of operation in which Discontinuous Transmission is not permitted.

Control Mobile Attenuation Code (CMAC)   A 3-bit field in the Control-Filler Message that specifies the maximum authorized power level for a mobile transmitting on an analog reverse control channels.

Convolution Code  	A type of error-correcting code. A code symbol can be considered as the convolution of the input data sequence with the impulse response of a generator function.

CRC  		See Cyclic Redundancy Code.

Cyclic Redundancy Code (CRC)  	A class of linear error detecting codes which generate parity check bits by finding the remainder of a polynomial division.

Data Burst Randomizer  The function that determines which power control groups within a frame are transmitted on the Reverse Traffic Channel when the data rate is lower than 9600 bps. The data burst randomizer determines, for each mobile station, the pseudo-random position of the transmitted power control groups in the frame while guaranteeing that every modulation symbol is transmitted exactly once.

DBc 	The ratio(in dB) of the sideband power of a signal, measured in a given bandwidth at a given frequency offset from the center frequency of the same signal, to the total inband power of the signal. For CDMA, the total inband power of the signal is measured in a 1.23MHz bandwidth around the center frequency of the CDMA signal.

DBm  	A measure of power expressed in terms of its ration (in dB) to one milliwatt.

DBm/Hz  	A measure of power spectral density. DBm/Hz is the power in one Hertz of bandwidth where power is expressed in units of dBm.

DBW  		A measure of power expressed in terns of its ratio (in dB) to one Watt.

Dedicated Control Channel 	An analog control channel used for the transmission of digital control information from either a base station or a mobile station.

Deinterleaving  	The process of unpermuting the symbols that were permuted by	the interleaver. Deinterleavering is performed on received symbols prior to decoding.

Digital Color Code(DCC) 	A digital signal transmitted by a base station on a forward analog control channel that is used to detect capture of a base station by an interfering mobile station.

Dim-and-Burst  	A frame in which primary traffic is multiplexed with either secondary traffic or signaling traffic.

Discontinuous Transmission (DTX)  	A mode of operation in which a mobile station transmitter autonomously switches between two transmitter power levels while the mobile station is in the conversation state on an analog voice channel.

Distance-Based Registration  An autonomous registration method in which the mobile station registers whenever it enters a cell whose distance from the cell in which the mobile station last registered exceeds a given threshold.

DTMF  		        See Dual Tone Multi Frequency.

Dual -Tone Multifrequency (DTMF)  	Signaling by the simultaneous transmission of two tones, one from a group of low frequencies and another from a group of high frequencies. Each group of frequencies consists of four frequencies.

Eb  			The energy of an information bit.

Ec/I0  	The ratio in (dB) between the pilot energy accumulated over one PN chip period (Ec) to the power spectral density in the received bandwidth (Io).

Effective Radiated Power (ERP)  	The transmitted power multiplied by the antenna gain referenced to a half wave dipole.

Electroluminescence	            A glowing effect used to backlight LCD panels.

Electronic Serial Number (ESN)  	A 32-bit number assigned by the mobile station manufacturer, uniquely identifying the mobile station equipment.

Encoder Tail Bits  	A fixed sequence of bits added to the end of a block of data to reset the convolutional encoder to a known state.

ERP  		See Effective Radiated Power.

ESN  		      See Electronic Serial Number.

Extended Protocol  	An optional expansion of the signaling message between the base station and mobile station to allow for the addition of new system features and operational capabilities.

FAA			The US Federal Aviation Administration

Fade Timer 	A timer kept by the mobile station as a measure of Forward Traffic Channel continuity. If the Fade timer expires, the mobile station drops the call.

FCC			The US Federal Communication Commission

FDMA		      Frequency Division Multiple Access

Flash 	An indication sent on an analog voice channel or CDMA Traffic Channel indicating that the user Directed the mobile station to invoke special processing.

FM			Frequency Modulation

Foreign NID Roamer  A mobile station operating in the same system (SID) but a different network (NID) than the one in which service was subscribed. See also Foreign SID Roamer and Roamer.

Foreign SID Roamer  A mobile station operating in a system (SID) other than the one from which service was subscribed. See also Foreign NID Roamer and Roamer.

Forward Analog Control Channel (FOCC) 	An analog voice channel used from a base station to a mobile station.

- Forward Analog Voice Channel (FVC) 	An analog voice channel used from a base station to a mobile station.

- Forward CDMA Channel 	A CDMA Channel form a base station to mobile stations. The Forward CDMA Channel contains one or more code channels that are transmitted on a CDMA frequency assignment using a Particular pilot PN offset. The code channels are associated with the Pilot Channel, Sync Channel, Paging Channels, and Traffic Channels. Forward CDMA Channel always carries a Pilot Channel and may carry up to one Sync Channel, up to seven Paging Channels, and up to 63 Traffic Channels, as long as the total number of channels, including the Pilot Channel, is no greater than 64.

Forward Traffic Channel 	A code channel used to transport user and signaling traffic from the base station to the mobile station. A basic timing interval in the system, for the Access Channel, Paging Channel, and Traffic Channel, a frame is 20 ms long. For the Sync Channel, a frame is 26.666…ms long.

Frame Category 	A classification of a received Traffic Channel frame based upon transmission data rate, the Frame contents (primary traffic, secondary traffic, or signaling traffic), and whether there are detected error in the frame.

Frame Offset 	A time skewing of Traffic Channel frames from System Time in integer multiples of 1.25 ms. The maximum frame offset is 18.75 ms.

Frame Quality Indicator  The CRC check applied to 9600 bps and 4800 bps Traffic Channel frames.

GHz			Gigahertz (109 Hertz)

Global Positioning System (GPS) 	A US government satellite system that provides location and time Information to users. See Navstar GPS Space segment/Navigation User interfaces ICD-GPS-200 for Specifications.

Half Frame 	A 10 ms interval on the paging Channel. Two half frames comprise a frame, the first half frame begins at the same time as the frame.

Handoff 	The of transferring communication with a mobile station from one base station to another.

Hard Handoff 	A handoff characterized by a temporary disconnection of the Traffic Channel. Hard handoffs Occur when the mobile station is transferred between disjoint Active Sets, the CDMA frequency assignment changes, the frame offset changes, or the mobile station is directed from a DMA Traffic Channel to an analog voice channel. See also Soft Handoff.

Hash Function 	A function used by the mobile station to select one out of N available resource. The hash function distributes the available resources                                  uniformly among a random sample of mobile stations.

HLR 			See Home Location Register.

Home Location Register (HLR) 	The location register to which a MIN is assigned for record purposes such as subscriber information.

Home System 	The cellular system in which the mobile station subscribes for service.

IB			Interface Box

Idle Handoff 	The act of transferring reception of the Paging Channel from one base station to another, when the mobile station is in the Mobile Station Idle State.

IEEE			Institute of Electrical and Electronic Engineers

IF			Intermediate Frequency

Implicit Registration    A registration achieved by a successful transmission of an	origination or page response on the Access Channel.

Interleaving 		The process of permuting a sequence of symbols.

kHz 			Kilohertz (103 Hertz).

ksps 			Kilo-symbols per second (103 symbols per second).

Layer 1 		See Physical Layer.

Layer 2 	Layer 2 provides for the correct transmission and reception of signaling messages, including partial duplicate detection. See also Layering and Layer 3.

Layer 3 	Layer 3 provides the control of the cellular telephone systems. Signaling messages originate and terminate at layer 3. See also Layering and Layer 2.

LED			Light Emitting Diode

LNA			Low Noise Amplifier

Local Control. 	An optional mobile station feature used to perform manufacturer-specific functions. A PN sequence with period 242-1 that is used for scrambling on the Forward CDMA Channel and spreading on the Reverse CDMA Channel. The long code uniquely identifies a mobile station on both the Reverse Traffic Channel and the Forward Traffic Channel. The long code provides limited privacy. The long code also separates multiple Access Channels on the same CDMA channel. See also Public Long Code and Private Long Code.

Long Code Mask	A 42-bit binary number that creates the unique identity of the long code. See also Public Long Code, Private Long Code, Public Long Code Mask, and Private Long Code Mask.

LSB 			Least significant bit.

Maximal Length Sequence (m-Sequence) 	A binary sequence of period 2n-1, with n  being a positive integer with no internal periodicities. A maximal length sequence can be generated by a tapped n-bit shift register with linear feedback.

mA			Milliamps (10-3 Amps)

Mcps 			Megachips per second (106 chips per second).

Mean Input Power 	The total received calorimetric power measured in a specified bandwidth at the antenna connector, including all internal and external signal and noise sources.

Mean Output Power   The total transmitted calorimetric power measured in a specified bandwidth at the antenna connector when the transmitter is active.

Message 	A data structure that conveys control information or application information. A message consists of a length field (MSG_LENGTH), a message body (the part conveying the information), and a CRC.

Message Body 	The part of the message contained between the length field MSG_LENGTH) and the CRC field.

Message Capsule 	A sequence of bits comprising a single message and padding. The	 padding always follows the message and may be of zero length.

Message CRC 	The CRC associated with a message.	See also Cyclic Redundancy check.

Message Field 	A basic named element in a message. A message field may consist of zero or more bits.

Message Record 	An entry in a message consisting of one or more field that repeats in the message.

MHz. 			Megahertz.(106 Hertz)

MIN 			See Mobile Station Identification Number.

Mobile Protocol Capability Indicator (MPCI) 	A 2-bit field used to indicate the mobile station’s capabilities.

Mobile Station 	A station in the Domestic Public Cellular Radio Telecommunications	            Service intended to be used while in motion or during halts at     unspecified points. Mobile station include portable units (e.g., handheld personal units) and units installed in vehicles.

Mobile Station Class	Mobile station classes define mobile station characteristics such as slotted operation and transmission power.

Mobile Station Identification Number (MIN) 	The 34-bit number that is a digital representation of the 10-digit directory telephone number assigned to a mobile station.

Mobile Station Originated Call 	A call originating from a mobile station.

Mobile Station Terminated Call 	A call received by a mobile station (not to be confused with a disconnect or call release).

Mobile Switching Center (MSC) 	A configuration of equipment that provides cellular radiotelephone service. Also called the Mobile Switching Office (MTSO.)

Modulation Symbol 	The output of the data modulator before spreading. On the Reverse Traffic Channel, 64-ary orthogonal modulation is used and six code symbol (when the data rate is 9600bps) or each repeated code symbol (when the data rate is less than 9600bps) is one modulation symbol.

Ms 			Millisecond

MS			Mobile Station

MSB 			Most significant bit

MSC 			See Mobile Switching Center.

MSO			Mobile Switching Office

Multiplex Option 	The ability of the multiplex sublayer and lower layer to be tailored to provide special capabilities. A multiplex option defines such characteristics as the frame format and the rate decision rules. See also Multiplex Sublayer.

Multiplex Sublayer 	One of the conceptual layers of the system that multiplexes and demultiplexes primary traffic, secondary traffic, and signaling traffic.

NAM 		      See Number Assignment Module.

Narrow Analog 	A type of voice channel that uses 10kHz channel spacing and audible signaling.

Neighbor Set          The set of pilots associated with the CDMA Channel that are probable candidates for handoff. Normally, the Neighbor Set consists of the pilots associated with CDMA Channel that cover geographical areas near the mobile station. See also Active Set, Candidate Set, and Remaining Set. A network is a subset of a cellular system, such as an area-wide cellular network, a private group of base stations, or a group of base stations set up to handle a special requirement. A network can be as small or as large as needed, as long as it is fully contained within a system. See also System.

Network Identification (NID) 	A number that uniquely identifies a network within a cellular system. See also System Identification.

NFPA			National Fire Protection Association

NID 			See Network Identification.

Non-Autonomous Registration 	A registration method in which the base station initiates registration. See also Autonomous Registration.

Non-Slotted Mode 	An operation mode of the mobile station in which the mobile station continuously monitors the Paging Channel when in the Mobile Station Idle State.

Ns 			Nanosecond.

NULL 		Not having any value.

Null Traffic Channel Data     One or more frames of 16 1’s followed by eight 0’s sent at the 1200 bps rate. Null Traffic Channel data is sent when no service option is active and no signaling message is being sent. Null Traffic Channel data serves to maintain the connectivity between the mobile station and the base station.

Number Assignment Module (NAM) 	A set of MIN-related parameters stored in the mobile station.

Numeric Information   Numeric information consists of parameters that appear as numeric fields in message exchanged by the base station and the mobile station and information used to describe the operation of the mobile station.

OLC 			See Overload Class (CDMA) or Overload Control (analog).

Optional Field 	A field defined within a message structure that is optionally to the message recipient.

Order 	A type of message that contains control codes for either the mobile station or the base station.

Ordered Registration   A registration method in which the base station orders the mobile station to send registration related parameters.

Overhead Message 	A message sent by the base station on the Paging Channel to communicate base-station-specific and system-wide information to mobile station.

Overload Class 	The means used to control system access by mobile stations, typically in emergency or other overload conditions. Mobile station are assigned one (or more) of sixteen overload classed, Access to the CDMA system can then be controlled on a per class basis by persistence values transmitted by the base station.

Overload Control (OLC) 	A means reverse analog control channel accesses by mobile stations. Mobile station are assigned one(or more) of sixteen control levels. Access is selectively restricted by a base station setting one or more OLC bits in the Overload Control Global Action Message.

Packet 	The unit of information exchanged between the service option applications of the base station and the mobile station.

Padding 	A sequence of bits used to fill from the end of a message to the end of a message capsule, typically to the end of the frame or half frame. All bits in the padding are 0.

Paging 	The act of seeking a mobile station when a call has been placed to that mobile station.

Paging Channel (Analog) 	See Analog Paging Channel.

Paging Channel (CDMA) 	A code channel in a Forward CDMA Channel used for transmission of control information and pages from a base station to a mobile station.

Paging Channel Slot     An 80ms interval on the Paging Channel. Mobile station operating in the slotted mode are assigned specific slots in which day monitor messages from the base station.

Parameter-Change Registration 	A registration method in which the mobile station registers when certain of its stored parameters change.

Parity Check Bits 	Bits added to a sequence of information bits to provide error detection, correction, or both.

Persistence 	A probability measure used by the mobile station to determine if it should transmit in a given Access Channel Slot.

Physical Layer 	The part of the communication protocol between the mobile station and the base station that is responsible for the transmission and reception of data. The physical layer in the transmitting station is presented a frame by the multiplex sublayer and transforms it into an over-the-air waveform. The physical layer in the receiving station transforms the waveform back into a frame and presents it to the multiplex sublayer above it.

Pilot Channel 	An unmodulated, direct-sequence spread spectrum signal transmitted continuously by each CDMA base station. The Pilot Channel allows a mobile station to acquire the timing of the Forward CDMA Channel, provides a phase reference for coherent demodulation, and provides a means for signal strength comparisons between base station for determining when to handoff.

Pilot PN Sequence 	A pair of modified maximal length PN sequences with period 215  used to spread the Foward CDMA Channel and the Reserve CDMA Channel. Different base station are identified by different pilot PN sequence offsets.

Pilot PN Sequence Offset Index 	The PN offset in units of 64 PN chips of a pilot, relative to the zero offset pilot PN sequence.

PLL			Phase Locked Loop

PN			Pseudo-noise

PN Chip 		One bit in the PN sequence.

PN Sequence 	Pseudonoise sequence. A periodic binary sequence.

Power Control Bit 	A bit sent in every 1.25ms interval on the Forward Traffic Channel to signal the mobile station to increase or decrease its transmit power.

Power Control Group    	A 1.25ms interval on the Forward Traffic Channel and the Reverse Traffic Channel. See also Power Control Bit.

Power-Down Registration 	Au autonomous registration method in which the mobile station registers on power up.

PPM 			Parts per million.

Preamble 		See Access Channel Preamble and Traffic Channel Preamble.

Primary CDMA Channel 	A CDMA Channel at a pre-assigned frequency assignment used by the mobile station for initial acquisition. See also Secondary CDMA Channel.

Primary Paging Channel (CDMA) The default code channel (code channel 1) assigned for paging on a CDMA Channel.

Primary Traffic 	The main traffic stream carried between the mobile station and the base station, supporting the active primary service option, on the Traffic Channel. See also Secondary Traffic, Signaling Traffic, and Service Option.

Private Long Code 	The long code characterized by the private long code mask. See also Long Code.

Private Long Code Mask 	The long code mask used to form the private long code. See also Public Long Code Mask and Long Code.

Public Long Code 	The long code characterized by the public long code mask.

Public Long Code Mask 	The long code mask used to form the private long code. The mask contains the ESN of the mobile station. See also Private Long Code Mask and Long Code.

Punctured Code 	An error-correcting code generated from another error-correcting code by deleting (i.e., puncturing) code symbols from the code output.

QPSK			Quadrature Phase Shift King

Quick Repeats 	Additional transmissions of identical copies of a message within a short interval to increase the probability that the message is received correctly.

Receive Objective Loudness Rating (ROLR) 	A perceptually weighted transducer gain            of telephone receivers relating electrical excitation from a reference generator to sound pressure at the earphone. The Receive Objective Loudness Rating is normally specified in dB relative to one Pascal per millivolt. See IEEE Standard 269-1992, IEEE Standard 661-1979, CCITT Recommendation P.76, and CCITT Recommendation P.79.

Registration 	The process by which a mobile station identifies its location and parameters to a base station.

Registration Zone 	A collection of one or more base stations treated as a unit when determining whether a mobile station should perform zone-based registration.

Release 	A process that the mobile station and base station use to inform each other of call disconnect. The set of all allowable pilot offsets as determined by PILOT_INC, excluding the pilot offsets of the pilots in the Active Set, Candidate Set, and Neighbor Set. See also Active Set, Candidate Set, and Neighbor Set.

Request 	A layer 3 message generated by either the mobile station or the base station to retrieve information, ask for service, or command an action.

Response 	A layer 3 message generated as a result of another message, typically a request.

Reverse Analog Control (RECC) 	The analog control channel used from a mobile       station to a base station.

Reverse Analog Voice Channel (RVC) 	The analog voice channel used from a mobile station to a base station.

Reverse CDMA Channel 	The CDMA Channel from the mobile station to the base station. From the base station’s perspective, the Reverse CDMA Channel is the sum of all mobile station transmissions on a CDMA frequency assignment.

Reverse Traffic Channel 	A Reverse CDMA Channel used to transport user and signaling traffic from a single mobile station to one or more base stations.

RF			Radio Frequency

Roamer 	A mobile station operating in a cellular system (or network) other than the one from which service was subscribed. See also Foreign NID Roamer and Foreign SID Roamer.

ROLR 		See Receive Objective Loudness Rating.

SAT 			See Supervisory Audio Tone.

Scan of Channels 	The procedure by which a mobile station examines the signal strength of each forward analog control channel.

SCI 			Synchronized Capsule Indicator bit.

Search Window 	The range of PN sequence offsets that a mobile station searches for	a pilot.

Secondary CDMA Channel 	A CDMA Channel at a pre-assigned frequency assignment used by the mobile station for initial acquisition. See also Primary CDMA Channel.

Secondary Traffic 	An additional traffic stream that can be carried between the mobile station and the base station on the Traffic Channel. See also Primary Traffic and Signaling Traffic.

Seizure Precursor 	The initial digital sequence transmitted by a mobile station to a base station on a reverse analog control channel.

Seizure Option 	A service capability of the system. Service options may be applications such as voice, data, or facsimile.

Shard Secret Data (SSD) 	A 128-bit pattern stored in the mobile station (in semi-permanent memory) and known by the base station. SSD is a concatenation of two 64-bit subsets: SSD_A, which is used to support the authentication procedures and SSD_B, which serves as one of the inputs to the process generating the encryption mask and private long code.

Short Message Services (SMS) 	A suite of services which include SMS Text Delivery, Digital Paging (i.e., Call Back Number – CBN), and Voice Mail Notification (VMN).

SID 			See System Identification.

Signaling Tone 	A 10kHz tone transmitted by a mobile station on an analog voice channel to: 1) confirm orders, 2) signal flash requests, and 3) signal release requests.

Signal Traffic 	Control message that are carried between the mobile station and base station on the Traffic Channel. See also Primary Traffic and Secondary Traffic.

Slot Cycle 	A periodic interval at which a mobile station operating in the slotted monitors the Paging Channel.

Slotted Mode 	An operation mode of the mobile station in which the mobile station monitors only selected slots on the Paging Channel when in the Mobile Station Idle State.

Soft Handoff 	A handoff occurring while the mobile station is in the Mobile Station Control on the Traffic Channel State. This handoff is characterized by commencing communications with a new base station on the same CDMA frequency assignment before terminating communications with the old base station. See also Hard Handoff.

SOM 			Start-of-Message Bit.

SPS 			Symbols per second.

STATUS IDENTIFICATION	An identification of certain characteristics of a mobile station. Classes are defined in Table 2.3.3-1. The following status information is used to describe mobile station operation when using the analog system.

- Serving-System Status 	Indicates whether a mobile station is turned to channels        associated with System A or System B.

- First Registration ID Status 	  A status variable used by the mobile station in   association with its processing of received Registration ID messages.

- First Location Area ID Status 	  A status variable used by the mobile station in association with its processing of received Location Area ID messages.

- Location Registration ID Status  A status variable used by the mobile station in   association with its processing of power-up registration and location-based registration.

- First Idle ID Status	A status variable used by the mobile station in  association with its processing of the Idle Task.

- Local Control Status	Indicates whether a mobile station must respond	to local control messages.

- Roam Status	Indicates whether a mobile station is in its home system.

- Termination Status	Indicates whether a mobile station must terminate the call when it is on an analog voice channel.

Supervisory Audio Tone (SAT) 	One of three tones in the 6 kHz region that is transmitted on the forward analog voice channel by a base station and transponder on the reverse analog voice channel by as mobile station.

Supplementary Digital Color Code (SDCC1, SDCC2) 	Additional bits assigned to increase the number of color codes from four to sixty four, transmitted on the forward analog control channel.

Symbol 	      See Code Symbol and Modulation Symbol.

Sync Channel 	Code channel 32 in the Forward CDMA Channel which transports the synchronization message to the mobile station.

Sync Channel Superframe 	An 80ms interval consisting of three Sync Channel frames  (each 26.666…ms in length).

System 	A system is a cellular telephone service that covers a geographic area such as a city, metropolitan region, country, or group of countries. See also Network.

System Time  	The time reference used by the system. System Time is synchronous to UTC time(except for leap seconds) and used the same time origin as GPS time. Offset by the propagation delay from the base station to the mobile station. See also Universal Coordinated Time.

TDMA		Time Division Multiple Access

Timer-Based Registration  A registration method in which the mobile station registers whenever a counter reaches a predetermined value. The counter is incremented an average of once per 80 ms period.

Time Reference  	A reference established by the mobile station that is synchronous with the earliest arriving multipath component used for demodulation.

TOLR  		See Transmit Objective Loudness Rating.

Traffic Channel  	A communication path between a mobile station and base station used for user and signaling traffic. The term Traffic Channel implies a Forward Traffic Channel and Reverse Traffic Channel pair. See also Forward Traffic Channel and Reverse Traffic Channel.

Traffic Channel Preamble  A sequence of all-zero frames that is sent at the 9600 bps rate by the mobile station on the Reverse Traffic Channel. The Traffic Channel preamble is sent during initialization of the Traffic Channel.

Transmit Objective Loudness Rating (TOLR)  	A perceptually weighted transducer gain of telephone transmitters relation sound pressure at the microphone to voltage at a reference electrical termination. It is normally specified in dB relative to one millivolt per Pascal. See IEEE Standard 269-1992, IEEE Standard 661-1979, CCITT recommendation P.76 , and CCITT Recommendation. P.79.

Unique Challenge-Response Procedure  	An exchange of information between a mobile station and a base station for the purpose of confirming the mobile station’s identity. The procedure is initiated by the base station and is characterized by the use of a challenge-specific random number (i.e., RANDU) instead of the random variable broadcast globally (RAND.)

Unique Random Variable (RANDU)  	A 24-bit random number generated by the base station in support of the Unique Challenge-Response procedure.

Universal Coordinated Time (UTC)  An internationally agreed-upon time scale maintained by the Bureau International de l’Heure(BIH) used as the time reference by nearly all commonly available time and frequency distribution systems i.e., WWW, WWVH, LORAN-C, Transit, Omega, and GPS.

UTC 			Universal Temps Coordiné.  See Universal Coordinated Time.

V			Volts

VCO			Voltage Controller Oscillator

VCTCXO 		Voltage Controlled Temperature Compensated Crystal Oscillator

Voice Channel  	See Analog Voice Channel.

Voice Mobile Attenuation Code (VMAC) 	  A 3-bit field in the Extended Address Word commanding the initial mobile power level when assigning a mobile station to an analog voice channel.

Voice Privacy  	The process by which user voice transmitted over a CDMA Traffic Channel is a afforded a modest degree of protection against eavesdropping over the air.

Walsh Chip  	The shortest identifiable component of a Walsh function. There are 2N Walsh chips in one Walsh function where N is the order of the Walsh function. On the Forward CDMA channel one Walsh chip equals 1/1.2288MHz, or 813.802 ns. On the Reverse CDMA Channel, one Walsh chip equals 4/1.2288 MHz, or 3.255 μs.

Walsh Function 	One of 2N time orthogonal binary functions (note that the functions are orthogonal after mapping 0 to 1 and 1 to –1.)

Zone-Based Registration 	An autonomous registration method in which the mobile station registers whenever it enters a zone that is not in the mobile station’s zone list.

㎲. 			Microsecond

APC			Automatic Power Control

BB			Baseband

BER			Bit Error Ratio

CLA			Cigar Lighter Adapter

DAC			Digital to Analog Converter

DCS1800			Digital Communication System

dBm			dB relative to 1 milliwatt

DSP			Digital Signal Processing

DTC			DeskTop Charger

EEPROM		Electrical Erasable Programmable Read-Only Memory

EL			Electroluminescence

ESD			Electrostatic Discharge

FPCB			Flexible Printed Circuit Board

GMSK		Gaussian Minimum Shift Keying

GPIB			General Purpose Interface Bus

GPRS		General Packet Radio Service

GSM			Global System for Mobile Communications

IPUI			International Portable User Identity

IF			Intermediate Frequency

LCD			Liquid Crystal Display

LDO			Low Drop Output

LED			Light Emitting Diode

LG-G510		LG GSM Phone

LGE			LG Electronics

OPLL		Offset Phase Locked Loop

PAM			Power Amplifier Module

PCB			Printed Circuit Board

PGA			Programmable Gain Amplifier

PLL			Phase Locked Loop

PSTN		Public Switched Telephone Network

RF			Radio Frequency

RLR			Receiving Loudness Rating

RMS			Root Mean Square

RTC			Real Time Clock

SAW			Surface Acoustic Wave

SIM			Subscriber Identity Module

SLR			Sending Loudness Rating

SRAM		Static Random Access Memory

STMR		Side Tone Masking Rating

TA			Travel Adapter

TDD			Time Division Duplex

TDMA		Time Division Multiple Access

UART		Universal Asynchronous Receiver/Transmitter

USB			Universal Serial Bus

VCO			Voltage Controlled Oscillator

VCTCXO		Voltage Control Temperature Compensated Crystal Oscillator

WAP		      Wireless A

Appendix

1. Component List

1.1 Accessories

1.2 Board

1.3 The Top of the Main PCB

1.4 The Bottom of the Main PCB

1.5 The Top of the Key PCB

1.6 The Bottom of the Key PCB

1.7 The Top of the FPCB

1.8 The Bottom of the FPCB

1.9 Mechanical Part

1. Component List

1.1 Accessories

| Description | LG Part No | Maker | QTY |
| --- | --- | --- | --- |
| CASE | MCEZ0001201 |  | 1 |
| COMPACT DISK | MCHZ0007401 |  | 1 |
| MANUAL,OPERATION | MMBB0173201 |  | 1 |
| SHEET,OPERATING | MSAB0011601 |  | 1 |
| SHEET,WARRENTY | MSAC0000401 |  | 1 |
| BATTERY PACK,LI-ION | SBPL0078701 | 엘지화학 | 2 |
| CHARGER,DESKTOP | SGCD0025701 | 동도전자 | 1 |
| DATA CABLE | SGDY0007901 | (주)니즈텍 | 1 |
| EAR PHONE/EAR MIKE SET | SGEY0004001 | 신우음향 | 1 |
| ADAPTOR,AC-DC | SSAD0016701 | 남경전자 | 1 |
| IC | EUSY0249501 | SANDISK CORP. | 1 |
| CONN,SOCKET | ENSY0015401 | ALPS | 1 |

1.2 Board

| Description | LG Part No | Maker | QTY |  |
| --- | --- | --- | --- | --- |
| PCB,FLEXIBLE | SPCY0049401 | PCB사업부(오산) | 1 |  |
| PCB,SIDEKEY | SPKY0018601 | PCB사업부(오산) | 1 |  |
| PCB,SIDEKEY | SPKY0018501 | PCB사업부(오산) | 1 |  |
| PCB,KEYPAD | SPEY0030401 | PCB사업부(청주) | 1 |  |
| PCB,MAIN | SPFY0093701 | PCB사업부(청주) |  | 1 |

1.3 The Top of the Main PCB

| Design No. | LG Part No | Description | Qty |
| --- | --- | --- | --- |
| C1100, C1102, C1103, C1111, C1121, C1204, C1205, C1268, C1269, C1308, C1309, C1311, C1318, C1319, C1320, C1322, C1372, C213, C248 | ECZH0000813 | CAP,CHIP,MAKER | 19 |
| C1101 | ECZH0000841 | CAP,CHIP,MAKER | 1 |
| C1110, C1122, C1123, C1124, C1209, C230, C328, C329, C346, C402, C403, C405, C407, C408 | ECZH0000838 | CAP,CHIP,MAKER | 14 |
| C1112 | ECTZ0005602 | CAP,TANTAL,CHIP,MAKER | 1 |
| C1114, C334, C335, C361, C371, C373, C375, C378, C380, C395, C409 | ECZH0003001 | CAP,CHIP,MAKER | 11 |
| C1115, C362 | ECTH0001903 | CAP,TANTAL,CHIP | 2 |
| C1117, C1126, C1194, C1208, C1220, C1248, C214, C215, C226, C227, C228, C233, C234, C235, C236, C237, C249, C250, C251, C252, C253, C254, C255, C256, C257, C259, C260, C262, C264, C265, C266, C267, C268, C310, C332, C333, C349, C352 | ECZH0001101 | CAP,CHIP,MAKER | 38 |
| C1129, C1143, C1144 | ECZH0000826 | CAP,CHIP,MAKER | 3 |
| C1132 | ELCH0004721 | INDUCTOR,CHIP | 1 |
| C1133, C1216, C1239, C1244, C1245 | ECZH0000830 | CAP,CHIP,MAKER | 5 |
| C1147, L1141, L1142, L1143 | ELCH0004710 | INDUCTOR,CHIP | 4 |
| C1152, C1197, C1215, C308, C309, C321, C365, C367, C392, C393 | ECZH0001109 | CAP,CHIP,MAKER | 10 |
| C1157, C1158 | ECZH0000805 | CAP,CHIP,MAKER | 2 |
| C1159, C1160 | ECZH0000844 | CAP,CHIP,MAKER | 2 |
| C1166, C1211, C1218, C1232, C1236, C1237, C1249, C210, C212, C229, C232, C238, C239, C240, C241, C243, C244, C246 | ECCH0002001 | CAP,CERAMIC,CHIP | 18 |
| C1175 | ECTH0002202 | CAP,TANTAL,CHIP | 1 |
| C1185 | ECZH0000845 | CAP,CHIP,MAKER | 1 |
| C1193, C301, C302, C343, C379, C381, C383, C387 | ECCH0005601 | CAP,CERAMIC,CHIP | 8 |
| C1195, C1206 | ECZH0026601 | CAP,CHIP,MAKER | 2 |
| C1196, C211, C219, C247, C311, C312, C316, C319, C354 | ECZH0001129 | CAP,CHIP,MAKER | 9 |
| C1198 | ECZH0026501 | CAP,CHIP,MAKER | 1 |
| C1199 | ECZH0001121 | CAP,CHIP,MAKER | 1 |

| Design No. | LG Part No | Description | Qty |
| --- | --- | --- | --- |
| C1201 | ECZH0001126 | CAP,CHIP,MAKER | 1 |
| C1202 | ECZH0001107 | CAP,CHIP,MAKER | 1 |
| C1207 | ECZH0000802 | CAP,CHIP,MAKER | 1 |
| C1242 | ECZH0000812 | CAP,CHIP,MAKER | 1 |
| C1250, C1251 | ECZH0000801 | CAP,CHIP,MAKER | 2 |
| C1254 | ECZH0001104 | CAP,CHIP,MAKER | 1 |
| C1355, C224, C225, C272, C347 | ECTH0002201 | CAP,TANTAL,CHIP | 5 |
| C1367 | ECZH0001002 | CAP,CHIP,MAKER | 1 |
| C217 | ECZH0001404 | CAP,CHIP,MAKER | 1 |
| C218 | ECZH0001115 | CAP,CHIP,MAKER | 1 |
| C220, C221, C222, C223 | ECZH0001202 | CAP,CHIP,MAKER | 4 |
| C242, C245 | ECZH0001208 | CAP,CHIP,MAKER | 2 |
| C303, C304, C305, C306, C307, C3400, C342, C3486, C388, C396 | ECZH0001128 | CAP,CHIP,MAKER | 10 |
| C330, C331 | ECZH0001105 | CAP,CHIP,MAKER | 2 |
| C336, C337 | ECZH0001125 | CAP,CHIP,MAKER | 2 |
| C338 | ECZH0001204 | CAP,CHIP,MAKER | 1 |
| C339, C340 | ECZH0001122 | CAP,CHIP,MAKER | 2 |
| C341 | ECCH0002004 | CAP,CERAMIC,CHIP | 1 |
| C344, C345 | ECTZ0004204 | CAP,TANTAL,CHIP,MAKER | 2 |
| C348, C359, C360, C363, C364, C366, C369, C370, C372, C376, C377, C394, C404 | ECZH0001418 | CAP,CHIP,MAKER | 13 |
| C3489, L1121, L1122 | ECZH0000818 | CAP,CHIP,MAKER | 3 |
| C350, C351 | ECZH0000821 | CAP,CHIP,MAKER | 2 |
| C357 | ECCH0002003 | CAP,CERAMIC,CHIP | 1 |
| C406 | ECTZ0005601 | CAP,TANTAL,CHIP,MAKER | 1 |
| CON412 | ENBY0019501 | CONNECTOR,BOARD TO BOARD | 1 |
| D201, D202, D310 | EDRY0000103 | DIODE,RECTIFIER | 3 |
| D311 | EDSY0010001 | DIODE,SWITCHING | 1 |
| D312 | EDSY0009901 | DIODE,SWITCHING | 1 |
| D415, D416 | EDTY0007501 | DIODE,TVS | 2 |
| F1100 | SFSY0021801 | FILTER,SAW | 1 |
| F1101 | SFSY0021701 | FILTER,SAW | 1 |
| F1102 | SFSY0015101 | FILTER,SAW | 1 |
| F1104 | SFSY0017301 | FILTER,SAW | 1 |
| Design No. | LG Part No | Description | Qty |
| F1109 | SFCY0000801 | FILTER,CERAMIC | 1 |
| F401, F402, F403 | SEVY0005801 | VARISTOR | 3 |
| L1103, L1105, L411 | ELCH0004727 | INDUCTOR,CHIP | 3 |
| L1106 | ELCH0004703 | INDUCTOR,CHIP | 1 |
| L1112 | ELCH0004709 | INDUCTOR,CHIP | 1 |
| L1113 | ECCH0000701 | CAP,CERAMIC,CHIP | 1 |
| L1119, L1120 | ELCH0004704 | INDUCTOR,CHIP | 2 |
| L1124, L1125, L1154 | ELCH0004701 | INDUCTOR,CHIP | 3 |
| L1128 | SFEY0006001 | FILTER,EMI/POWER | 1 |
| L1144, L1195 | ELCH0004722 | INDUCTOR,CHIP | 2 |
| L1153, R1110, R1114 | ELCH0004706 | INDUCTOR,CHIP | 3 |
| L1178 | ELCH0004712 | INDUCTOR,CHIP | 1 |
| L1179 | ELCH0004718 | INDUCTOR,CHIP | 1 |
| L310, L317 | SFBH0000903 | FILTER,BEAD,CHIP | 2 |
| L314, L315, L316 | ELCP0008001 | INDUCTOR,SMD,POWER | 3 |
| Q1102 | EQBA0000602 | TR,BJT,ARRAY | 1 |
| Q311 | EQFP0001302 | TR,FET,P-CHANNEL | 1 |
| Q313 | EQBN0004701 | TR,BJT,NPN | 1 |
| Q318 | EQBP0004102 | TR,BJT,PNP | 1 |
| Q319 | EQBP0006601 | TR,BJT,PNP | 1 |
| Q414 | EQBP0002402 | TR,BJT,PNP | 1 |
| R1101, R213, R219, R220, R227, R228, R231, R233, R312, R339, R458 | ERHZ0000405 | RES,CHIP,MAKER | 11 |
| R1102 | ERHZ0000505 | RES,CHIP,MAKER | 1 |
| R1103, R1104, R1133, R1139, R387 | ERHY0003301 | RES,CHIP | 5 |
| R1105, R1130 | ERHZ0000402 | RES,CHIP,MAKER | 2 |
| R1111, R1115 | ERHZ0000458 | RES,CHIP,MAKER | 2 |
| R1112, R1117, R1152, R243, R320, R329, R341, R344, R345, R380, R392 | ERHZ0000401 | RES,CHIP,MAKER | 11 |
| R1113, R1201 | ERHZ0000428 | RES,CHIP,MAKER | 2 |
| R1120 | ERHZ0000483 | RES,CHIP,MAKER | 1 |
| R1131 | ERHZ0000435 | RES,CHIP,MAKER | 1 |
| R1132, R343 | ERHZ0000530 | RES,CHIP,MAKER | 2 |
| R1134, R1137, R337 | ERHZ0000203 | RES,CHIP,MAKER | 3 |

| Design No. | LG Part No | Description | Qty |
| --- | --- | --- | --- |
| R1135 | ERHZ0000257 | RES,CHIP,MAKER | 1 |
| R1149 | ERHY0003201 | RES,CHIP | 1 |
| R1150 | ERHZ0003203 | RES,CHIP,MAKER | 1 |
| R1163, R1164 | ERHZ0003202 | RES,CHIP,MAKER | 2 |
| R1202, R1203 | ERHZ0000531 | RES,CHIP,MAKER | 2 |
| R210 | ERHZ0000217 | RES,CHIP,MAKER | 1 |
| R211 | ERHZ0000231 | RES,CHIP,MAKER | 1 |
| R212 | ERHZ0000285 | RES,CHIP,MAKER | 1 |
| R214, R357 | ERHZ0000490 | RES,CHIP,MAKER | 2 |
| R215 | ERHZ0000437 | RES,CHIP,MAKER | 1 |
| R216, R325, R326, R461, R476 | ERHZ0000404 | RES,CHIP,MAKER | 5 |
| R217 | ERHZ0000205 | RES,CHIP,MAKER | 1 |
| R222, R223, R224, R225 | ERHZ0000443 | RES,CHIP,MAKER | 4 |
| R226 | ERHZ0000494 | RES,CHIP,MAKER | 1 |
| R229, R230 | ERHZ0000433 | RES,CHIP,MAKER | 2 |
| R236 | SEVY0003601 | VARISTOR | 1 |
| R315 | ERHZ0000465 | RES,CHIP,MAKER | 1 |
| R330, R331, R364, R385 | ERHZ0000486 | RES,CHIP,MAKER | 4 |
| R332, R333, R335, R336 | ERHZ0002401 | RES,CHIP,MAKER | 4 |
| R334 | ERHZ0000449 | RES,CHIP,MAKER | 1 |
| R338 | ERHZ0000406 | RES,CHIP,MAKER | 1 |
| R342 | SEVY0004101 | VARISTOR | 1 |
| R346 | ERHZ0000204 | RES,CHIP,MAKER | 1 |
| R361, R490, R491, R492, R493, R494 | ERHZ0000493 | RES,CHIP,MAKER | 6 |
| R370 | ERHZ0000213 | RES,CHIP,MAKER | 1 |
| R386 | ERHZ0003701 | RES,CHIP,MAKER | 1 |
| R388, R389 | ERHZ0000488 | RES,CHIP,MAKER | 2 |
| R390 | ERHZ0003901 | RES,CHIP,MAKER | 1 |
| R421 | ERHZ0000498 | RES,CHIP,MAKER | 1 |
| R457, R459, R464 | SEVY0003602 | VARISTOR | 3 |
| U1101 | SFAD0000901 | FILTER,SEPERATOR,SP3T | 1 |
| U1103 | EUSY0219401 | IC | 1 |
| U1104 | SMPY0008301 | PAM | 1 |
| U1110 | EQFP0003501 | TR,FET,P-CHANNEL | 1 |

| Design No. | LG Part No | Description | Qty |
| --- | --- | --- | --- |
| U1111 | SFAY0004101 | FILTER,SEPERATOR | 1 |
| U1116 | EUSY0181702 | IC | 1 |
| U1119 | EUSY0196901 | IC | 1 |
| U1124, U1126 | SFAC0000101 | FILTER,SEPERATOR,SPDT | 2 |
| U1135 | EUSY0257501 | IC | 1 |
| U1150 | EUSY0256901 | IC | 1 |
| U210 | EUSY0217901 | IC | 1 |
| U211 | EUSY0067201 | IC | 1 |
| U212 | EUSY0170901 | IC | 1 |
| U310 | EUSY0159601 | IC | 1 |
| U312 | EUSY0249701 | IC | 1 |
| U313, U319 | EUSY0188601 | IC | 2 |
| U314 | EUSY0142501 | IC | 1 |
| U315 | EUSY0160001 | IC | 1 |
| U317 | EUSY0203701 | IC | 1 |
| U320 | EUSY0166201 | IC | 1 |
| U410 | EUSY0155301 | IC | 1 |
| VR1100, VR1107, VR1108, VR1109 | SEVY0006401 | VARISTOR | 4 |
| VR1101, VR1102, VR1106, VR314 | SEVY0003801 | VARISTOR | 4 |
| X210 | EXRY0001101 | RESONATOR | 1 |
| X310 | EXXY0016601 | X-TAL | 1 |

1.4 The Bottom of the Main PCB

| Design No. | LG Part No | Description | Qty |
| --- | --- | --- | --- |
| C1105, C1173 | ECCH0000701 | CAP,CERAMIC,CHIP | 2 |
| C1119 | ECZH0000845 | CAP,CHIP,MAKER | 1 |
| C1127, C427, C428 | ECZH0000816 | CAP,CHIP,MAKER | 3 |
| C1130, C1138, C1151, C1162, C1168, C1227, C1228, C1235, C1270, C1300, C1302, C1303, C1304, C1305, C1313, C1314, C1315, C1316, C1317, C1323, C1324, C1325, C1326, C1329, C1330, C1336, C1339, C1342, C1343, C1344, C1345, C1350, C1357, C1358, C1360, C1375, C275, C445, L1196 | ECZH0000813 | CAP,CHIP,MAKER | 39 |
| C1134, C1213, C1222, C1224, C1229, C1230, C1233, C1246, C1338 | ECZH0000838 | CAP,CHIP,MAKER | 9 |
| C1135, C1140, C1189, C1192, C1234, C1240, C1243, C1247, C231 | ECCH0002001 | CAP,CERAMIC,CHIP | 9 |
| C1137, L1148 | ELCH0004708 | INDUCTOR,CHIP | 2 |
| C1139, C1150 | ECZH0001418 | CAP,CHIP,MAKER | 2 |
| C1141, C1148, C1161, C1163, C1167, C1187, C1223, C1272, C1273, C1274, C1276, C1301, C1341, C1346, C3488 | ECZH0001109 | CAP,CHIP,MAKER | 15 |
| C1145, C1153, C1170 | ECZH0000819 | CAP,CHIP,MAKER | 3 |
| C1149, C1221 | ECZH0000802 | CAP,CHIP,MAKER | 2 |
| C1165, C1181, C1231, C368 | ECZH0001101 | CAP,CHIP,MAKER | 4 |
| C1169, C1278 | ECZH0000803 | CAP,CHIP,MAKER | 2 |
| C1171 | ECZH0000841 | CAP,CHIP,MAKER | 1 |
| C1172, C355, C356, C422, C423, C424, C426, C430, C433, C434, C435, C436, C448 | ECZH0001129 | CAP,CHIP,MAKER | 13 |
| C1174, C1184, C1186, C1327, C1340, C1348, C1359 | ECZH0000812 | CAP,CHIP,MAKER | 7 |
| C1176 | ECZH0000809 | CAP,CHIP,MAKER | 1 |
| C1177, C1180, C1212 | ECZH0000818 | CAP,CHIP,MAKER | 3 |
| C1179, C1376, C425, C431, C432, C446, C449, C451 | ECZH0001128 | CAP,CHIP,MAKER | 8 |
| C1183 | ECZH0000817 | CAP,CHIP,MAKER | 1 |
| C1188 | ECFD0000102 | CAP,FILM,MPP | 1 |
| C1190, C1191, C1266 | ECZH0000806 | CAP,CHIP,MAKER | 3 |
| C1210, C1217, C1226, C1328, C1352 | ECZH0003001 | CAP,CHIP,MAKER | 5 |
| C1214 | ECZH0000804 | CAP,CHIP,MAKER | 1 |
| C1219 | ECZH0001501 | CAP,CHIP,MAKER | 1 |
| Design No. | LG Part No | Description | Qty |
| C1225 | ECTZ0003605 | CAP,TANTAL,CHIP,MAKER | 1 |
| C1238 | ECZH0000833 | CAP,CHIP,MAKER | 1 |
| C1241, C1337, C1356 | ECZH0001121 | CAP,CHIP,MAKER | 3 |
| C1263, C1264, C1271, C216, C3401, C3402, C3403, C397, C398, C399 | ECZH0000830 | CAP,CHIP,MAKER | 10 |
| C1267 | ECZH0000828 | CAP,CHIP,MAKER | 1 |
| C1279, L1108 | ELCH0004704 | INDUCTOR,CHIP | 2 |
| C1299 | ECZH0000824 | CAP,CHIP,MAKER | 1 |
| C1321, C1333, C1354, C1370, C1371, C1373, C273 | ECCH0005601 | CAP,CERAMIC,CHIP | 7 |
| C1335 | ECTH0002201 | CAP,TANTAL,CHIP | 1 |
| C1374 | ECTH0001903 | CAP,TANTAL,CHIP | 1 |
| C274, C322, C324 | ECZH0003501 | CAP,CHIP,MAKER | 3 |
| C323, C325, C326, C327 | ECCH0005602 | CAP,CERAMIC,CHIP | 4 |
| C374, C382 | ECZH0001215 | CAP,CHIP,MAKER | 2 |
| C410 | ECTZ0003601 | CAP,TANTAL,CHIP,MAKER | 1 |
| C447, C450, C452 | ECZH0001213 | CAP,CHIP,MAKER | 3 |
| C453 | ECZH0000810 | CAP,CHIP,MAKER | 1 |
| CON311, CON312 | ENSY0009401 | CONN,SOCKET | 2 |
| CON410 | ENBY0024501 | CONNECTOR,BOARD TO BOARD | 1 |
| CON411 | ENRY0002401 | CONNECTOR,I/O | 1 |
| CON414 | ENBY0001802 | CONNECTOR,BOARD TO BOARD | 1 |
| CON415 | ENJE0004101 | CONN,JACK/PLUG,EARPHONE | 1 |
| CON416 | ENSY0015801 | CONN,SOCKET | 1 |
| D410 | EDTY0001901 | DIODE,TVS | 1 |
| D413 | EDTY0006201 | DIODE,TVS | 1 |
| D414 | EDRY0000103 | DIODE,RECTIFIER | 1 |
| DP1101 | SFAY0002801 | FILTER,SEPERATOR | 1 |
| DP1102 | SDDY0001802 | DUPLEXER,DCN | 1 |
| DP1103 | SDPY0002901 | DUPLEXER,PCS | 1 |
| F1103 | SFSY0018201 | FILTER,SAW | 1 |
| F1105 | SFSY0024901 | FILTER,SAW | 1 |
| F1106 | SFSY0016801 | FILTER,SAW | 1 |
| F1108 | SFSY0025301 | FILTER,SAW | 1 |
| F404, F405, F406, F407, F408 | SEVY0005501 | VARISTOR | 5 |
| L1107 | ELCH0004709 | INDUCTOR,CHIP | 1 |

| Design No. | LG Part No | Description | Qty |
| --- | --- | --- | --- |
| L1109, L1139, L1140 | ELCH0004721 | INDUCTOR,CHIP | 3 |
| L1110 | ELCH0004729 | INDUCTOR,CHIP | 1 |
| L1111 | ELCH0004714 | INDUCTOR,CHIP | 1 |
| L1114, L1117 | ECZH0000822 | CAP,CHIP,MAKER | 2 |
| L1116, L1147, L1150, L1159, L1181, L1182, L1184, L1187, L1188, L1192, L1197, L1198, L311, R234, R235 | SFBH0000903 | FILTER,BEAD,CHIP | 15 |
| L1123, L1135, L1137, L1138 | ELCH0004713 | INDUCTOR,CHIP | 4 |
| L1129 | ELCH0001431 | INDUCTOR,CHIP | 1 |
| L1130 | ELCH0004705 | INDUCTOR,CHIP | 1 |
| L1131 | ELCH0004716 | INDUCTOR,CHIP | 1 |
| L1132, L1151 | ELCH0004712 | INDUCTOR,CHIP | 2 |
| L1133 | ELCH0004701 | INDUCTOR,CHIP | 1 |
| L1136 | ELCH0004710 | INDUCTOR,CHIP | 1 |
| L1145, L1149 | ELCH0004718 | INDUCTOR,CHIP | 2 |
| L1152 | ELCH0004717 | INDUCTOR,CHIP | 1 |
| L1185, L1186 | ELCH0002708 | INDUCTOR,CHIP | 2 |
| L1189, L1190 | ELCH0002713 | INDUCTOR,CHIP | 2 |
| L410 | SFEY0006001 | FILTER,EMI/POWER | 1 |
| Q1100, Q1101 | EQBA0000602 | TR,BJT,ARRAY | 2 |
| Q1103 | EQFP0001301 | TR,FET,P-CHANNEL | 1 |
| Q1104, Q317, Q410, Q417 | EQBN0004703 | TR,BJT,NPN | 4 |
| R1106 | ERHZ0000206 | RES,CHIP,MAKER | 1 |
| R1107 | ERHZ0003202 | RES,CHIP,MAKER | 1 |
| R1108, R1109 | ERHZ0000325 | RES,CHIP,MAKER | 2 |
| R1116 | ERHZ0000404 | RES,CHIP,MAKER | 1 |
| R1118, R1119, R1143, R1148 | ERHZ0000490 | RES,CHIP,MAKER | 4 |
| R1121, R1122, R1177, R1228, R1236, R1237, R232, R419, R420, R424, R503, R504, R510 | ERHZ0000401 | RES,CHIP,MAKER | 13 |
| R1124, R1125, R1136 | ERHY0003301 | RES,CHIP | 3 |
| R1126, R1129 | ERHZ0000483 | RES,CHIP,MAKER | 2 |
| R1128 | ERHZ0000512 | RES,CHIP,MAKER | 1 |
| R1140, R1179, R310, R313, R410 | ERHZ0000405 | RES,CHIP,MAKER | 5 |
| R1141 | ERHZ0000201 | RES,CHIP,MAKER | 1 |
| R1142, R1145, R1180 | ERHZ0000402 | RES,CHIP,MAKER | 3 |
| R1144 | ERHZ0000486 | RES,CHIP,MAKER | 1 |

| Design No. | LG Part No | Description | Qty |
| --- | --- | --- | --- |
| R1146 | ERHZ0000437 | RES,CHIP,MAKER | 1 |
| R1147 | ERHZ0000278 | RES,CHIP,MAKER | 1 |
| R1151 | ERHZ0003203 | RES,CHIP,MAKER | 1 |
| R1178, R1212, R1213, R360, R363 | ERHZ0000485 | RES,CHIP,MAKER | 5 |
| R1194 | ERHZ0000522 | RES,CHIP,MAKER | 1 |
| R1195, R1196 | ERHY0003501 | RES,CHIP | 2 |
| R1208, R1216 | ERHZ0000212 | RES,CHIP,MAKER | 2 |
| R1210, R1211, R1214, R1215, R412, R413, R414, R415, R416, R417, R418, R423 | ERHZ0000484 | RES,CHIP,MAKER | 12 |
| R1223, R1226, R431 | ERHZ0000454 | RES,CHIP,MAKER | 3 |
| R1224, R1225, R314 | ERHZ0000438 | RES,CHIP,MAKER | 3 |
| R1231 | ERHZ0000250 | RES,CHIP,MAKER | 1 |
| R323, R327, R411, R497 | ERHZ0000444 | RES,CHIP,MAKER | 4 |
| R348, R349 | ERHZ0000475 | RES,CHIP,MAKER | 2 |
| R351, R352, R353, R354, R355, R356, R359, R362, R422, R430, R433, R435, R436, R437, R479, R481, R482, R483, R484, R486, R487, R502 | SEVY0004401 | VARISTOR | 22 |
| R384, VR310, VR311, VR312, VR313 | SEVY0005101 | VARISTOR | 5 |
| R438 | SEVY0004101 | VARISTOR | 1 |
| R455 | ERHZ0000507 | RES,CHIP,MAKER | 1 |
| R456, R480 | ERHZ0000407 | RES,CHIP,MAKER | 2 |
| R460, R495, R496 | SEVY0003602 | VARISTOR | 3 |
| R485, R499, R500, R513, R514 | SEVY0003601 | VARISTOR | 5 |
| R498 | ERHZ0000406 | RES,CHIP,MAKER | 1 |
| R501 | SEVY0000901 | VARISTOR | 1 |
| R505, R506, R507, R508 | SEVY0004301 | VARISTOR | 4 |
| R509 | ERHZ0000517 | RES,CHIP,MAKER | 1 |
| U1100 | ENWY0003301 | CONN,RF SWITCH | 1 |
| U1102 | EUSY0102801 | IC | 1 |
| U1105 | EUSY0133701 | IC | 1 |
| U1106 | EUSY0188001 | IC | 1 |
| U1107 | SMPY0009101 | PAM | 1 |
| U1108 | SMPY0005401 | PAM | 1 |
| U1109 | EXSC0009201 | VCO | 1 |
| U1112 | SFAY0004101 | FILTER,SEPERATOR | 1 |

| Design No. | LG Part No | Description | Qty |
| --- | --- | --- | --- |
| U1113 | EXSC0006501 | VCO | 1 |
| U1114 | EXSK0003201 | VCTCXO | 1 |
| U1115 | EUSY0133801 | IC | 1 |
| U1118 | SFAY0005801 | FILTER,SEPERATOR | 1 |
| U1120 | EUSY0255901 | IC | 1 |
| U1122 | SFAC0000101 | FILTER,SEPERATOR,SPDT | 1 |
| U1136 | EUSY0257401 | IC | 1 |
| U1138 | EQBA0004101 | TR,BJT,ARRAY | 1 |
| U1139, U1151 | EUSY0256901 | IC | 2 |
| U1140 | EUSY0256801 | IC | 1 |
| U1141 | EUSY0116003 | IC | 1 |
| U1146, U1147 | EUSY0255801 | IC | 2 |
| U1149, U1152 | EUSY0116004 | IC | 2 |
| U216 | EUSY0241402 | IC | 1 |
| U311 | EUSY0183302 | IC | 1 |
| U318 | EUSY0277101 | IC | 1 |
| U321 | MCBA0010301 | CAN,SHIELD | 1 |
| U411, U413, U414 | EUSY0232812 | IC | 3 |
| U412 | EUSY0166201 | IC | 1 |
| U416, U417 | SFEY0006702 | FILTER,EMI/POWER | 2 |
| VR1103 | SEVY0006401 | VARISTOR | 1 |

1.5 The Top of the Key PCB

| Design No | LG Part No | Description | Qty |
| --- | --- | --- | --- |
| VR7 | SEVY0003801 | VARISTOR | 1 |
| R10,R11,R12,R2,R3,R4,R5,R6,R7,R8,R9 | SEVY0003601 | VARISTOR | 11 |

1.6 The Bottom of the Key PCB

| Design No | LG Part No | Description | Qty |
| --- | --- | --- | --- |
| CON1 | ENBY0020101 | 20PIN-CONNECTOR | 1 |
| C1 | ECZH0000844 | CHIP-CAPACITOR | 1 |
| C2 | ECZH0001108 | CHIP-CAPACITOR | 1 |
| C3 | ECZH0001501 | CHIP-CAPACITOR | 1 |
| U1 | EUSY0250101 | LAMP-DRIVER-IC | 1 |
| VR2,VR3,VR5,VR6 | SEVY0003801 | VARISTOR | 4 |
| R13 | SEVY0004101 | VARISTOR | 1 |
| D1 | EDNY0010401 | ZENER-DIODE | 1 |
| R1 | ERHZ0000507 | RESISTOR | 1 |
| L1 | ELCP0006807 | POWER-INDUCTOR | 1 |
| D2,D3 | EDTY0007501 | TVS-DIODE | 2 |

1.7 The Top of the FPCB

| Design No | LG Part No | Description | Q'ty |
| --- | --- | --- | --- |
| C1, C2, C3, C4 | ECZH0001129 | CHIP-CAPACITOR | 4 |
| C8 | ECZH0000812 | CHIP-CAPACITOR | 1 |
| L1, L2 | ELCH0001425 | CHIP-INDUCTOR | 2 |
| L3 | SFBH0000903 | BEAD-INDUCTOR | 1 |
| LED1 | EDLM0005801 | FLASH-LED | 1 |
| LED2, LED3 | EDLH0006001 | BLUE-LED | 2 |
| R12, R13 | ERHZ0000493 | RESISTOR | 2 |
| R14 | ERHZ0000206 | RESISTOR | 1 |
| R4, R5 | ERHZ0000702 | RESISTOR | 2 |
| R6, R7 | ERHZ0000402 | RESISTOR | 2 |
| VR1, VR2, VR3, VR4 | SEVY0003801 | VARISTOR | 4 |
| VR5, VR6, VR7, VR8, VR9 | SEVY0004401 | VARISTOR | 5 |
| X1 | EXSY0020501 | OSCILLATOR | 1 |

1.8 The Bottom of the FPCB

| Design No | LG Part No | Description | Q'ty |
| --- | --- | --- | --- |
| CON1 | ENBY0024401 | 64PIN-CONNECTOR | 1 |
| CON2 | ENBY0022401 | 50PIN-CONNECTOR | 1 |
| CON3 | ENBY0019101 | 24PIN_CON_FEMALE | 1 |
| R15, R18 | ERHZ0000493 | RESISTOR | 2 |

1.9 Mechanical Part

| No | Part No | Description | QTY | Maker |
| --- | --- | --- | --- | --- |
| 1 | MFBB0011501 | FILTER,RECEIVER | 1 |  |
| 2 | MGAZ0021001 | GASKET | 1 |  |
| 3 | MMAA0000301 | MAGNET,SWITCH | 1 |  |
| 4 | MPBG0032501 | PAD,LCD | 1 |  |
| 5 | MPBJ0024201 | PAD,MOTOR | 1 |  |
| 6 | MPBN0020201 | PAD,SPEAKER | 1 |  |
| 7 | MPBZ0087901 | PAD | 1 |  |
| 8 | MTAD0037501 | TAPE,WINDOW | 1 |  |
| 9 | MBJC0015601 | BUTTON,FUNCTION | 1 |  |
| 10 | MCJJ0033001 | COVER,FOLDER(UPPER) | 1 |  |
| 11 | MICZ0009101 | INSERT | 4 |  |
| 12 | MDAD0014901 | DECO,CAMERA | 1 |  |
| 13 | MDAE0029501 | DECO,FOLDER(UPPER) | 1 |  |
| 14 | MFBC0016101 | FILTER,SPEAKER | 1 |  |
| 15 | MGAZ0018301 | GASKET | 1 |  |
| 16 | MLCD0004601 | LENS,CAMERA | 1 |  |
| 17 | MLCE0004001 | LENS,FLASH | 1 |  |
| 18 | MPBJ0024101 | PAD,MOTOR | 1 |  |
| 19 | MPBN0024601 | PAD,SPEAKER | 1 |  |
| 20 | MPBQ0022001 | PAD,LCD(SUB) | 1 |  |
| 21 | MPBT0015201 | PAD,CAMERA | 1 |  |
| 22 | MTAA0078501 | TAPE,DECO | 1 |  |
| 23 | MTAA0078401 | TAPE,DECO | 1 |  |
| 24 | MTAB0067001 | TAPE,PROTECTION | 1 |  |
| 25 | MTAB0067101 | TAPE,PROTECTION | 1 |  |
| 26 | MTAE0022201 | TAPE,WINDOW(SUB) | 1 |  |
| 27 | MTAZ0063801 | TAPE | 1 |  |
| 28 | MBIB0003201 | BUSHING,HINGE | 1 |  |
| 29 | MBIB0003601 | BUSHING,HINGE | 1 |  |
| 30 | MBJL0021501 | BUTTON,SIDE | 1 |  |
| 31 | MBJL0021601 | BUTTON,SIDE | 1 |  |
| 32 | MCJK0038801 | COVER,FRONT | 1 |  |
| 33 | MICA0018101 | INSERT,FRONT | 6 |  |
| 34 | MDAG0012601 | DECO,FRONT | 1 |  |
| 35 | MDAJ0008601 | DECO,HINGE | 2 |  |

| No | Part No | Description | QTY | Maker |
| --- | --- | --- | --- | --- |
| 36 | MTAA0086401 | TAPE,DECO | 1 |  |
| 37 | MFBD0009601 | FILTER,MIKE | 1 |  |
| 38 | MPBZ0087601 | PAD | 1 |  |
| 39 | MSGB0010301 | STOPPER,HINGE | 1 |  |
| 40 | MTAA0086201 | TAPE,DECO | 1 |  |
| 41 | MTAB0084701 | TAPE,PROTECTION | 2 |  |
| 42 | MTAB0079801 | TAPE,PROTECTION | 1 |  |
| 43 | MDAL0005301 | DECO,WINDOW | 1 |  |
| 44 | MTAA0086101 | TAPE,DECO | 1 |  |
| 45 | MTAB0066701 | TAPE,PROTECTION | 1 |  |
| 46 | MWAC0052101 | WINDOW,LCD | 1 |  |
| 47 | GMEY0005901 | SCREW MACHINE,BIND | 4 |  |
| 48 | MCCH0050801 | CAP,SCREW | 4 |  |
| 49 | MCCZ0012901 | CAP | 1 |  |
| 50 | MHFD0011601 | HINGE,FOLDER | 1 |  |
| 51 | MLAZ0038303 | LABEL | 1 |  |
| 52 | MWAF0026801 | WINDOW,LCD(SUB) | 1 |  |
| 53 | MPFZ0017101 | PLATE | 1 |  |
| 54 | MTAZ0072801 | TAPE | 1 |  |
| 55 | MGAZ0021101 | GASKET | 1 |  |
| 56 | MGAZ0021201 | GASKET | 1 |  |
| 57 | MGAZ0021301 | GASKET | 2 |  |
| 58 | MIDZ0073901 | INSULATOR | 1 |  |
| 59 | MIDZ0074001 | INSULATOR | 1 |  |
| 60 | MIDZ0076201 | INSULATOR | 1 |  |
| 61 | MIDZ0076701 | INSULATOR | 1 |  |
| 62 | MPBQ0025401 | PAD,LCD(SUB) | 1 |  |
| 63 | MPBZ0099401 | PAD | 1 |  |
| 64 | SJMY0002601 | VIBRATOR,MOTOR | 1 | 엘지이노텍 |
| 65 | SUVT0005801 | TWO-WAY MODE SPEAKER | 1 | 기린텔레콤 |
| 66 | SVCY0005901 | CAMERA | 1 | PANASONIC |
| 67 | SVLY0023401 | LCD | 1 | TOSHIBA MATSUSHITA DISPLAY TECHNOLOGY CO.,LTD. |
| 68 | GMEY0011401 | SCREW MACHINE,BIND | 1 |  |

| No | Part No | Description | QTY | Maker |
| --- | --- | --- | --- | --- |
| 69 | MCCC0023901 | CAP,EARPHONE JACK | 1 |  |
| 70 | MCCE0018901 | CAP,RECEPTACLE | 1 |  |
| 71 | MCCG0002301 | CAP,MULTIMEDIA CARD | 1 |  |
| 72 | MCIA0014201 | CONTACT,ANTENNA | 1 |  |
| 73 | MCJN0035301 | COVER,REAR | 1 |  |
| 74 | MICZ0023401 | INSERT | 1 |  |
| 75 | MGAD0099601 | GASKET,SHIELD FORM | 1 |  |
| 76 | MGAD0093901 | GASKET,SHIELD FORM | 1 |  |
| 77 | MGAD0088801 | GASKET,SHIELD FORM | 1 |  |
| 78 | MIDZ0073101 | INSULATOR | 1 |  |
| 79 | MLAA0021405 | LABEL,APPROVAL | 1 |  |
| 80 | MLAN0000604 | LABEL,QUALCOMM | 1 |  |
| 81 | MLAR0004501 | LABEL,WARNING | 1 |  |
| 82 | MLEA0023201 | LOCKER,BATTERY | 1 |  |
| 83 | MLEE0000401 | LOCKER,CARD READER | 2 |  |
| 84 | MPBZ0106501 | PAD | 1 |  |
| 85 | MPBZ0087801 | PAD | 1 |  |
| 86 | MSAZ0027101 | SHEET | 1 |  |
| 87 | MSAZ0027201 | SHEET | 1 |  |
| 88 | MSDC0002001 | SPRING,LOCKER | 1 |  |
| 89 | SNMF0017301 | ANTENNA,MOBILE,FIXED | 1 | 이엠더블유안테나 |
| 90 | GMEY0005901 | SCREW MACHINE,BIND | 6 |  |
| 91 | MCCF0024301 | CAP,MOBILE SWITCH | 1 |  |
| 92 | MCCH0057801 | CAP,SCREW | 1 |  |
| 93 | MCCH0070801 | CAP,SCREW | 1 |  |
| 94 | MCJA0019001 | COVER,BATTERY | 1 |  |
| 95 | MTAB0095201 | TAPE,PROTECTION | 1 |  |
| 96 | MIDZ0076801 | INSULATOR | 1 |  |
| 97 | MKAA0013401 | KEYPAD,DIAL | 1 |  |
| 98 | MKAC0006701 | KEYPAD,FUNCTION | 1 |  |
| 99 | MLAA0021401 | LABEL,APPROVAL | 1 |  |
| 100 | MPHY0001001 | PROTECTOR | 2 |  |
| 101 | ADCA0034401 | DOME ASSY,METAL | 1 |  |
| 102 | MGAZ0022001 | GASKET | 1 |  |
| 103 | MIDZ0073201 | INSULATOR | 1 |  |

| No | Part No | Description | QTY | Maker |
| --- | --- | --- | --- | --- |
| 104 | MIDZ0073801 | INSULATOR | 1 |  |
| 105 | MPBL0004001 | PAD,MSM | 1 |  |
| 106 | SMZY0010801 | MODULE,ETC | 1 | 3 LASER TECHNOLOGY |
| 107 | MBFZ0022401 | BRACKET | 1 |  |
| 108 | MFEA0007501 | FRAME,SHIELD | 1 |  |
| 109 | MIDZ0071301 | INSULATOR | 1 |  |
| 110 | MIDB0000801 | INSULATOR,RECEIVER | 1 |  |
| 111 | MIDZ0076901 | INSULATOR | 1 |  |
| 112 | MIDZ0077101 | INSULATOR | 1 |  |
| 113 | SBCL0001004 | BATTERY,CELL,LITHIUM | 1 | TECHTUIT |
| 114 | SUMY0008902 | MICROPHONE | 1 | 보성전자 |
| 115 | MLAZ0038301 | LABEL | 1 |  |