Laboratory Exercises A630

Introduction

This document has been prepared to provide learning technicians the hands-on- training  skills necessary to effectively troubleshoot the GSM LCA Platform cellular transceiver. The reader of this document should have already participated in a level 3 technical training session that discussed the theory of operation, testing, and programming of the GSM phones. If such training has not been taken yet, contact your local training coordinator to schedule a technical training session for the GSM phone.

It is required that the reader of this document is already familiar with test equipment related to radio communications. This includes the following:

	Oscilloscope

	Multimeter

	Spectrum Analyzer

	Cellular communications Analyzer

Hardware requirements

In order to successfully complete the laboratory exercises, the following hardware will be needed.

| GSM A630 |  |
| --- | --- |
| Torx #6 driver | USB Data Cable (SKN6311A) |
| Plastic disassembly tool (6680388B67) | Battery (SNN5704A) |
| RF Connector (2887920K01) | Charger (SPN4716B) |
| Test SIM |  |

If this hardware is not available, contact your local parts administrator for ordering information.

Lab 1 (GSM Measurements)

Call Processing Setup

Use the following procedures for call processing.

- Install the test SIM and battery on phone.

- Connect the RF cable from the phone to the test instrument

- Setup up the test box for GSM, DCS, or PCS Signalling

- Set Broadcast Channel (BCH) to 20 (GSM900),  700 (DCS), or 661 (PCS)

- Set Broadcast channel level to -85dBm

- Set Traffic Channel (TCH) to 62 (GSM900) or 512 (DCS/PCS)

- Set Traffic channel level to -85dBm

- Set Power control level to 10

- Wait until the phone indicates a receive signal

- Dial a number from the phone and press the send button.

- The phone is now connected.

GSM900 (Setup channels for GSM900)

- Read Burst Average Power. 		Power _____________dBm

- Verify Burst Output Shape.  		Pass / Fail 	(circle one)

- Read RMS Phase Error			Phase Error ____________	deg

- Read Frequency Error			Frequency Error___________Hz

- Read RX Quality@-105dBm		RX Quality___________

(set BCH level to -105dBm)

GSM1900 (Setup channels for PCS)

- Read Burst Average Power. 		Power _____________dBm

- Verify Burst Output Shape.  		Pass / Fail 	(circle one)

- Read RMS Phase Error			Phase Error ____________	deg

- Read Frequency Error			Frequency Error___________Hz

- Read RX Quality@-105dBm		RX Quality___________

(set BCH level to -104dBm)

Handover

Perform the following handover procedures.

| Band | From |  | To |  |
| --- | --- | --- | --- | --- |
|  | Traffic Channel | Power Control | Traffic Channel | Power Control |
| GSM900 | 62 | 5 | 124 | 19 |
| DCS | 512 | 0 | 885 | 15 |
| PCS | 512 | 0 | 810 | 15 |

Pass / Fail	 (circle one)

Pass / Fail	 (circle one)

Pass / Fail	 (circle one)

GSM Manual Measurements

GSM TX Power

Use RadioComm to send the following commands.

- SUSPEND

- CP_MODE - Click on 900 (GSM)

- LOAD_SYN - Enter 62 (GSM) and then click Set

- SET_RF_PWR - Enter 5 (GSM) and then click Set

- CARRIER - Select 06 and then click ON

Read GSM TX Power		TX Power______________________dBm

GSM900 RSSI

Setup test instrument with following parameters,

Broadcast Channel (BCH): 		  20

Broadcast Channel (BCH) Level:	-105 dBm

Use RadioComm to send the following commands.

- SUSPEND

- CP_MODE - Click on 900

- SCMP - Enter Channel 20 and click INIT

- RQEP - Click Execute

Read GSM900 RSSI		RSSI_____________________dBm

Lab 2 (Handset Test Commands)

To enter Hand Test Menue “ Menue PHTCMDK ”

Main LCD Backlight test

Enter handset test mode and issue following commands

54			Suspend

62*1*1		Enable Main LCD Backlight

62*1*0		Disable Main LCD Backlight

Verify operation					Pass / Fail	(circle one)

Keypad Backlight test

Enter handset test mode and issue following commands

54			Suspend

62*0*1		Enable Keypad Backlight

62*0*0		Disable Keypad Backlight

Verify operation					Pass / Fail	(circle one)

Vibrator Test

Enter handset test mode and issue following commands

54			Suspend

3*0*1			Enable Vibrator

3*0*0			Disable Vibrator

Verify operation					Pass / Fail	(circle one)

Handset Mic/Speaker test

Enter handset test mode and issue following commands

54			Suspend

6*2*2*0*0		Enable internal mic and handset speaker

4*4*1  		Enable VOCODER loopback at Enhanced Full Rate

Verify audio loopback					Pass / Fail	(circle one)

Mono Headset Mic/Speaker test

Enter handset test mode and issue following commands

54			Suspend

6*4*6*0*0		Enable headset mic and headset speaker

4*4*1  		Enable VOCODER loopback at Enhanced Full Rate

Verify audio loopback					Pass / Fail	(circle one)

Display Color test

Enter handset test mode and issue following commands

54			Suspend

55*2*014		Eight Color Box Pattern

Verify edges (uniform/smooth)			Pass / Fail	(circle one)

Display Linearity test

Enter handset test mode and issue following commands

54			Suspend

55*2*005		Grey Scale Block

Verify smooth transition from dark to light		Pass / Fail	(circle one)

Display Flicker test

Enter handset test mode and issue following commands

54			Suspend

55*2*006	Horizontal Zebra Line

Verify that no noticable flicker exists			Pass / Fail	(circle one)

Display Pixel Defect test

Enter handset test mode and issue following commands

54			Suspend

55*2*001		All pixels on (all white)

55*2*000		All pixels off (all black)

Verify that less than 3 pixel defects			Pass / Fail	(circle one)

Lab 3 (RadioComm)

Melody Speaker Test

Enter handset test mode and issue following commands

- SUSPEND

- ALERT – Clasic Enable

Verify operation					Pass / Fail	(circle one)

Vibrator Test

Enter handset test mode and issue following commands

- SUSPEND

- AUD_CTRL – Click Vib ON

Verify operation					Pass / Fail	(circle one)

Handset Mic/Speaker test

Enter handset test mode and issue following commands

- SUSPEND

- AUD_PATH – Select internal mic and internal speaker. Click Set

- AUD_LPB - Select Enhanced Full Rate and click Vocoder

Verify audio loopback					Pass / Fail	(circle one)

Mono Headset Mic/Speaker test

Enter handset test mode and issue following commands

- SUSPEND

- AUD_PATH – Select boom mic and boom spkr right. Click Set

- AUD_LPB - Select Enhanced Full Rate and click Vocoder

Verify audio loopback					Pass / Fail	(circle one)

Display Backlight test

Enter handset test mode and issue following commands

- SUSPEND

- LEDS – Main Display

Verify operation					Pass / Fail	(circle one)

Display Color test

Enter handset test mode and issue following commands

- SUSPEND

- TST_DISP(Predefined) - Select Eight Color Box and click Set

Verify edges (uniform/smooth)			Pass / Fail	(circle one)

Display Linearity test

Enter handset test mode and issue following commands

- SUSPEND

- TST_DISP(Predefined) - Select Grey Scale Block and click Set

Verify smooth transition from dark to light		Pass / Fail	(circle one)

Display Flicker test

Enter handset test mode and issue following commands

- SUSPEND

- TST_DISP(Predefined) - Select Horizontal Zebra and click Set

Verify that no noticable flicker exists			Pass / Fail	(circle one)

Display Pixel Defect test

Enter handset test mode and issue following commands

- SUSPEND

- TST_DISP(Predefined) - Select All pixels on (all white) and click Set

- TST_DISP(Predefined) - Select All pixels off (all black) and click Set

Verify that less than 3 pixel defects			Pass / Fail	(circle one)

Keypad Backlight test

Enter handset test mode and issue following commands

- SUSPEND

- LEDS - Select Keypad to enable. Deselect Keypad to disable.

Verify operation					Pass / Fail	(circle one)

Bluetooth TX test

- SUSPEND

- BLUETOOTH - select parameter 84 and click execute

- BLUETOOTH -  select 81 and click execute.

- BLUETOOTH - select parameter 01 and enter 2DFC0129 in the “TO Radio” field. Click Execute.

Using a spectrum analyzer verify that a 2441MHz signal is present. Use a RF probe to sniff the strongest signal around the “7” key of the internal keypad.

Bluetooth TX power		_____________dBm

Lab 4 (GSM Troubleshooting)

RX Troubleshooting

Setup

BCH: 62 -> 947,4MHz

BCH Level: -30dBm

Measurements

Measure GSM_RF/ANTENNA1 at C184	_______________dBm

Measure GSM_RF/RX_GSM at Z151	_______________dBm

Measure RX_CP at C122			_______________Vdc

TX Troubleshooting

Setup

TX Channel: 62 -> 902,4MHz

TX Power Level: 5

Measurements

Measure TX_MOD at R111			_____________Hz

Measure GSM_RF/TX_IN_LB  No Part or Test point available

Measure TX_CP at C112			____________Vdc

Measure AOC_DRIVE at L194		____________Vdc

Measure GSM_RF/ANTENNA1 at C184	___________dBm

Setup

TX Channel: 124 -> 914,8MHz

TX Power Level: 15

Measurements

Measure TX_MOD at R111			_____________Hz

Measure GSM_RF/TX_IN_LB  No Part or Test point available

Measure TX_CP at C112			____________Vdc

Measure AOC_DRIVE at L194		____________Vdc

Measure GSM_RF/ANTENNA1 at C184	___________dBm

Switching Signals Checking

Check following signals at RX and TX

Setup TX:

TX Channel: 62

TX Power Level: 15

Setup RX

RX Channel: 62

| TX | Frequency Hz | Voltage Vdc |
| --- | --- | --- |
| EUROB/US |  |  |
| EXC/EN |  |  |
| RX/EN |  |  |
| TX/EN |  |  |
| RX |  |  |
| EUROB/US |  |  |
| EXC/EN |  |  |
| RX/EN |  |  |
| TX/EN |  |  |

RX Audio Troubleshooting

Setup

Generate 1kHz tone using RadioComm

- SUSPEND

- AUD_PATH – Select internal speaker and internal mic. Click Set

- AUD_TN_GEN – select 1 Freq, enter 1000 in Freq 1 field, click Start Tones

Measurements

Measure HAND_SPKR+ at C444	_____________kHz, ___________Vpp

Baseband Troubleshooting

Measure 26MHz at C203		__________MHz, ___________Vpp

Measure CLK_32.768K at C302	__________kHz, __________Vpp

| Name | Voltage Vdc |
| --- | --- |
| VBUCK |  |
| VBUST |  |
| RF_5V_REG |  |
| RF_REG |  |
| REF_REG |  |
| NEP_IO_REG |  |
| IO_REG |  |
| B+ |  |
| AUD_REG |  |