Contact Information : TEL +82-31-670-0665 / FAX +82-31-670-0662

| ( Written by : Brady Dang) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| DOC No. | : | VK-SVC-02-005 |  |  |  |
| Issue date | : | 2003. 09. 24. |  |  |  |
| To | : | HMM |  |  |  |
| Cc | : | Every SVC Center |  |  |  |
| Subject | : | Hitachi LCD Module component Change notice |  |  |  |

1. Purpose

Hitachi LCD Module component Change notice

2．Changed Part

1) Changed component : Temperature compensation IC in Hitachi LCD module for VG100/CG100. (Remarked in red circle)

2) Changed Part Information

|  | VK CODE | Part name | Changed Component |  |
| --- | --- | --- | --- | --- |
|  |  |  | Name | Component code |
| Old | 36600011 | LCD MODULE (HITACHI) | Temperature compensation IC | S-8120A |
| New | 36600011 | LCD MODULE (HITACHI) | Temperature compensation IC | #### S-8120C |

Notice : VK code would NOT be changed.

3) Reason

The old Temperature compensation IC (S-8120A) is end of production.

Location of Temperature compensation IC

3. Old & New Hitachi LCD Module discrimination

|  | Old HITACHI LCD module | New HITACHI LCD module |
| --- | --- | --- |
| Temperature compensation IC | Old | New |
| Distinguishing remark on LCD connector | None, R, E, H, 2, etc. | “X” mark Only |
| Remarking Location |  |  |
| S/W Name | VG100~_H_~R CG100~_H_~R | VG100~_HZ~R CG100~_HZ~R |

4. Caution

1) Different S/W for each old & New Hitachi LCD Module

- Hitachi LCD Module is used for VG100 & CG100 (Not for VG-207)

- Each Old & New Hitachi LCD Module has its own software.

That is, software for Old & New Hitachi LCD Module is different.

2) How to distinguish S/W

- Old Hitachi LCD module S/W (Indicator : H)

  - VG100~_H_~R

  - Example : VG107RU_HZ1.04R

- New Hitachi LCD module S/W (Indicator : HZ)

  - VG100~_HZ~R

  - Example : VG107RU_H_01.3R

3) S/W version check

- Using HIDDEN STRING(Engineering mode)

  - *#85*364#*

  - Press “3” to select “S/W Version”

5．Notice

If improper S/W is loaded into old or new Hitachi LCD Module handset,

handset’s RF characteristic would be bad depending on the temperature.

For example, call drop, weak T/X, R/X signal, No network search etc.