Disassemble and Assemble Guide

# Disassemble Process

- Preparation: Put on anti-static wrist strap, finger covers and anti-static suit. No stain such as finger print is allowed on LCD during the process.

- Tools: T4 screwdriver, hydronium gun, narrow knife, tweezers, and so on.

## Remove Back Housing

        - Tear off the disassemble-proof label from the back housing. Use a T4 screwdriver to unscrew the four screws on the back housing.

- Disassembel-proof label

- Fixing screw

        - Disjoin the housing front assembly from the housing bottom assembly near the I/O connector. Do not lift the housing bottom assembly too high, because there are one annular hook on the top and one clip on each side.

Disassemble at this direction.

        - Slide the housing bottom assembly in the antenna direction to release the top annular hook until the housing bottom assembly is totally removed.

Remove the rear case at this direction.

- The removed housing bottom assembly is as shown in the following figure:

## Break Off FPC Connector

- Break off the FPC connector that is connected with the LCD drive board.

  Note:

To avoid the FPC pad falling off, use a special tool to pry the long side to remove the FPC connector. Do not pry any other part.

- Take off FPC connector this way

- Cover for FPC connector

## Remove Main Board

- Remove the main board from the housing bottom assembly.

## Remove Keyboard

Remove the keyboard with a tweezers.

## Remove Hinge

- Push with a wimble through the hole on the hinge while pulling out the flip with light force until removing the hinge.

  Note:

Hold the handset tightly and pulling out the flip with light force.

- Hinge hole

## Remove Flip Front Assembly

- Do as follows:

        - Use a straight screwdriver or a narrow knife to remove the covers of two screws.

        - Use a cross screwdriver to remove the two screws.

        - Use a special tool to rip between the flip front assembly and the flip rear assembly. Then turn the flip front assembly about the hinge to separate the two.

## Remove LCD, SPEAKER, Receiver, and Motor

- During the removal, notice the connection wires of LCD, SPEAKER, receiver, and motor.

# Assembly Process