Welcome to TCL	2

1. Basic Operations	3

2. Phone Layout	6

3. Specifications	9

4. Getting Started	10

5. Making and Answering Calls	12

6. Features Menu	16

6.1 Message Functions	16

6.2 Recent Calls	19

6.3 Phone Book	20

6.4 Clock	21

6.5 Audio Profiles	21

6.6 Settings	22

6.8 Games	25

7. Entry MethodS	26

8. Safety and Precautions	27

Appendix I : TROUBLESHOOT	28

Appendix II : Abbreviations and Explanations	30

Note: TCL reserves the right to change or improve the product design, specifications, and other descriptions in this manual without prior notice. TCL also reserves the final explanation right for this manual.

.

# WELCOME TO TCL

Congratulations on purchasing a TCL cellular phone!

This User’s Manual introduces you to your new TCL cellular phone through simple and easy-to-follow instructions. You are probably anxious to start using your phone. We do encourage you to immediately go to Section 1: Basic Operations. It contains all the information you need to quickly activate your phone, set up and much more. Then, familiarize yourself with our features by going through Section 6: Features Menu. Enjoy!

Thank you for choosing TCL.

# 1. BASIC OPERATIONS

##### This section walks you through the step-by-step basics of setting up service for your TCL Cellular Phone. Once you have completed this section, you are ready to explore the advanced features of your new phone.

##### Switching ON

To turn your phone on, press and hold the POWER key.

Once the phone is ON, it will automatically search the network. When the handset is registered in a network, the network name will be displayed on the screen. You are now ready to make and answer calls. If the screen displays “Please insert SIM card”, switch the phone off by pressing and holding on POWER. Remove the battery and install your SIM card in the designated slot. If a password or PIN code is required, enter the correct password. For security purpose, the code will not be visible as you type. Then, press the Left Soft Key for confirmation. After the phone is unlocked, the handset will indicate the name of the registered network and the signal intensity. .

Please contact your network supplier if you do not know your PIN code..  The handset default password is 1234.

##### Switching OFF

To turn off the handset, press and hold the POWER key.

The handset will be switched off after the log-off animation.  Do not attempt to switch off the phone by directly removing the battery. This may result in SIM card memory loss.

##### Communication Control Keys

- CALL/END CALL Keys: Press the CALL key to answer an incoming call and to make a call after entering the phone numbers. . In standby mode, press the CALL key to display the latest call records (all answered, missed and dialed calls, maximum 30 numbers). Press CALL twice to dial a record selected. Press END CALL key to end a call or to return to standby mode from menu. END CALL button also serves as power on/off button when pressed and held for 2 seconds..

- DIRECTION Keys and Right/Left Soft Keys: . In standby mode, press the DIRECTION key to navigate the main menu or browse menu items. Press the Left Soft Key to access the phone book directly. . In menu, . the functions of the both Soft Keys are displayed at the bottom line of the main screen.

- Numeric Keys: Keys from  to, , and are used to dial numbers and enter passwords, English letters, symbols, characters etc. They can also be used to select menus. Press and hold  to  keys for more than 2 seconds to quickly dial the phone number correspondingly to the shortcut key.. (Please see Section 6.3 for shortcut key setup).

Main Screen Display Icons

There are 8 icons in total on the first line of the screen:.

Antenna.

Signal Intensity.

Keypad lock..

Alarm clock is enabled.

Voice mailbox. Icon flashing means message storage is full..

Battery indicator.

During the handset operational process, there are some symbols to help you operate the phone:

Ring.

Ring after vibrate.

Vibrate.

Mute mode.

Access the phone book function..

You have unread messages or info message (or also known as cell   broadcasts)..

Receiving Prompt Modes.

With a built-in vibrating motor and a buzzer, the handset can offer the following prompt modes for receiving calls:.

Ring.

Vibrate.

Ring after vibrate.

Mute mode.

##### STK Function

The handset supports STK (SIM Tool Kit) function. The menu items are pre-determined by the SIM card network provider. . The function is only available when the STK service is provided by the network and when your SIM card supports the STK function. In this case, the STK menu will be added automatically to the end of the handset's main menu accordingly.

.

IMEI (International Mobile Equipment Identify)

.Switch off the handset and unload the battery, you can see the bar code label at the back of the handset. . The code stored in the handset can be displayed by entering “* # 06 #” in standby mode. It is required by law to have this identification code . identification printed on your phone. The IMEI code for each handset is unique globally.

Enhanced Full Rate Coding (EFR)

This handset has Enhanced Full Rate Coding function. It does not need to be preset. The handset can detect the network system automatically.  If this service is provided by your network, the handset can automatically activate the function to supply you with good sound quality and high receptivity.

# 2. PHONE LAYOUT

The handset has 18 keys, a display screen, a built-in vibrating motor, and a recharge jack.

The Front Schematic Layout.

The Back Schematic Layout

.

# 3. SPECIFICATIONS

| SPECIFICATIONS | PARAMETERS |
| --- | --- |
| Size | 81x45x16 mm |
| Weight | 63g |
| Volume | 55 cc |
| Bands | Tri-band 900/1800MHz |
| LCD | 128x64 pixels |
| System Connector | DC jack, earphone jack |
| SIM | STK (SIM Tool Kit) |
| Battery | Standard Li-Ion 550 mAh |
| Battery Life | Talk time 4 hours, Standby time 150 hours |
| Short Message Service (SMS) | Point to Point SMS |
| T9 | Predictive text input |
| Phone Book | 200 entries |
| Speed Dial | 8 locations |
| Call Features | Call Waiting, Call Forwarding |
| Call List | 10 missed, 10 dialed, 10 received |
| Keypad Lock | Yes |
| Languages | English |
| Game | Yes |
| Calculator | Yes |

# 4. GETTING STARTED

##### Charging the battery

Your TCL Q515 is equipped with a Lithium Ion (Li-Ion) battery.

Charge the battery before using the phone. A new battery will reach its nominal capacity after fully charged and discharged a few times later. In order to use the battery at its optimum mode, we recommend discharging the battery completely once a month. To do so, simply keep your phone on until the battery runs out. To charge the battery, attach it to the phone, then, connect the supplied AC adapter. Make sure the direction sign is facing upward. When the handset is charging, the symbol of the battery indicator increases gradually from right to left. The handset can be charged under both ON or OFF status. When the battery is fully charged, the battery indicator shows full grid. To remove from the power supply, unplug the handset from the cable by pressing the buttons on both sides.

Notes on Charging:

- When charging the battery, avoid:

  - Charging for long periods of time. It will reduce battery life.

  - Charging in temperatures below 32°F (0°C) or over 104°F (40°C).

  - Charging the battery in direct sunlight or in places with high humidity (such as bathroom).

- Do not expose the battery to temperatures above 140°F (60°C). For example, do not leave the battery in direct sunlight on the dashboard of a vehicle.

- It is normal for the battery to become slightly warm during charging or using.

- Be sure to switch off the handset before unloading the battery, otherwise, it will shorten the lifespan of the handset and the data saved may be lost.

Checking the Battery

The battery lasts approximately as indicated in the following table before it needs to be recharged. Battery life may vary depending on conditions of use.

| Type of battery | Standby time | Talking time |
| --- | --- | --- |
| Li-ion battery | 50-150 hours | 150-250 minutes |

When the battery is low, the handset will give off a warning tone and display “Low Battery” on the screen. When the power is completely drained, the handset will automatically switch off.

Note: To ensure safety usage, do not tamper with the battery. We also suggest only using the battery and charger supplied with your TCL phone.

# 5. MAKING AND ANSWERING CALLS

##### Making Calls

Use the number buttons to enter the numbers. If you enter the wrong digit, press the Right Soft Key to back up or press and hold to clear the entire entry. When a call is established, you will hear a connecting prompt tone. Call meter or call charge will be displayed on the screen. If a call is not established, there might be one of several reasons:

- LINE BUSY… means the recipient is busy or the network has no spare line. If the auto-redial function is preset, the handset will redial automatically.

- REFUSED… means the call has been blocked by the receiving party.

##### Dial with Shortcut Key

The handset provides 9 shortcut keys. You can customize the phone numbers corresponding to the keys (see Phone Book for detail information). In this case, you can make a quick call by press and hold  to  key in standby mode..

Note: For speed dialing, you have to press and hold key (1-9) for over 2 seconds. You need to define your shortcut key beforehand. Please refer to Section 6.3...

##### View Last Numbers

The handset can save up to 30 latest communicating numbers. Press and hold the CALL key.  in standby mode can display the latest numbers. . You can press the CALL key again to directly dial the number by selecting the number required with the DIRECTION keys.

.

##### Send DTMF

When DTMF (Dual Tone Multiple Frequency) function is set, the numeric key you entered from the keypad  and  key will be transmitted to the recipient as DTMF tone..

##### Mute Function

##### During a call, press the Right Soft Key to mute the microphone. You will see “Call Mute” at the lower right corner of the screen. . In this situation, the recipient cannot hear you, but you can still hear him (the earpiece is still on), re-press the Right Soft Key to enable the microphone. Press END CALL button to hang up.

##### Call Waiting

You can only use this function if you subscribe to the value-added service of call waiting. The following function menu will occur when pressing the Left Soft Key during a call.

- Volume: To adjust sound volume. .

- Hold: To hold an ongoing call, then, make another call..

- Hang up: To end the current call..

- Mute: To shut off the microphone..

- On/Off DTMF: To enable/disable function for calling another extension..

- Main menu: To access phone book and message menu..

- Conference call: To access conference call..

Press the DIRECTION key to switch between the two calls. If the current call is put on hold, another function menu can occur by pressing the Left Soft Key during the call:.

- Volume: To adjust sound volume..

- Switch: To hold/activate the call..

- Hang up: To end the current call..

- Mute: To shut off the microphone..

- On /Off DTMF: To enable/disable function for calling another extension..

- Main menu: To access phone book and message..

- Conference call: To access conference call..

Press END CALL key to hang up all calls..

Read Short Message.

When new messages arrive, the screen will display the list. You can access the list to read the message(s)..

##### Missed Calls

When you miss your incoming calls, the main screen will display the list. You can access the list to view the details of unanswered call(s)..

Cell Broadcast.

When cell broadcasts arrive, the screen will display the list. You can access the list to read the cell broadcast(s).

##### Connect to Voice Mailbox

The handset allows you to forward the incoming calls to a voice mailbox. The number of the voice mailbox center must be preset. You can preset your Voice Mailbox in Messages Menu under Voicemail.

###### Answer Calls

When an incoming call arrives, you will be prompted in the following ways: ringing, vibrating or backlight flashing, according to your handset settings..

- To answer a call, press the CALL key..

- Press the END CALL key to reject the call..

If you subscribe to Caller ID function, the phone number of the incoming call will be displayed on the screen. If the number has already been saved in the phone book, the name will also be displayed on the screen.

# 6. FEATURES MENU

## 6.1 Message Functions.

This handset supports the short message function. . A message is sent to the short message center of the network, then, to the targeted user by the network. . If the targeted user does not log on or fails to receive the message (i.e. full memory), the network will temporally store the message for a designated period of time..  To transmit and receive short messages, you may have to subscribe to the service from your network supplier in advance unless it’s already included in your service. N. N Not only does this phone support the normal short message service, but also the enhanced Short Message Service (EMS). This means that you are able to receive messages with attachments (e.g. inserted pictures, ringing, and animations, etc.).  If the memory of short message is not full, a message will be saved directly in the SIM card. When icon  is flashing on the top row of the main screen, it means the memory is full. When viewing or editing a message, press the DIRECTION key to scroll the message pages, press and hold the direction key to move the cursor on a character basis.

.

【Inbox】: You can view the received messages in Inbox..

- Edit: To access short message edit box..

- Delete: To delete the current message(s)..

- Forward: To forward the message to another person..

- File: To save the received message in the Favorite. .

- Text reply: To reply to the received message in text..

- Voice reply: To reply to the received message in voice.

.

【Outbox】: You can read the message(s) here before you send it..

【Write messages】: You can create new message(s) in this menu..

- Complete: To handle the message..

- Send: To transmit message(s). . You can input the phone number directly, or press the DIRECTION key to access the phone book list to select the number..

- Save: To save the edited message in the SIM card..

- File: To save the edited message in the Favorite of your handset.

- Options:: There are a few options you can choose from when writing messages.

  - T9 Options: MultiTap, T9 Symbol, T9 Number, T9 English. These options are available whenever you input information.

  - Insert Item: Picture, Animation, Melody, Message, Emoticon.

  - Apply Format: Italic, Strikeout. .

【Favorite】: You can save messages in the handset..

Under the root directory, press the CALL key to access the directory and the subfolder list. You can create, save, delete, edit, rename, etc. a file or a folder as required.   After accessing the Favorite, press the Left Soft Key for four menus:.

- Info: To provide type of files, total and residual space, etc..

- Delete: To delete the selected files or folders..

- Rename: To rename a selected file or folder..

- New folder: To create a new folder under the current directory.. .

【Message settings】: You can preset message parameters..

- Message Center: To send a message, you must enter the number of the local short message center in advance (provided by the network supplier.)..

- Message Lifetime: The valid retention time of a message (i.e. one hour, two days, etc.). .

- Message Format: To select the type of message (i.e. Normal, Fax, etc.).

【Voicemail】

- You can call the voice mailbox to listen to a message. . To call a voice mailbox, you have to preset the number of the voice mailbox center in advance. This number is provided by your network supplier.

- Get V-mail: To retrieve your voice mail.

- Voicemail settings: The setting of the voice mailbox.

  - Box Numbers: Your handset is preset with two mailboxes - Home Mbox and Roam Mbox.

  - Alert Match: You can also set an alert for a specified phone numbers..

【Info Message】: Info message, also known as, cell broadcast is the information service to all GSM users in a specific locale sent by the network supplier, such as weather forecast, transportation conditions, etc..

- Read: To read a cell broadcast in this menu..

- Settings: To preset cell broadcast. .

  - Language: To select a language for cell broadcast display..

  - Set Info Subject: To preset the band of cell broadcast.

    - User Defined: To define your own subject.

    - Current List: To view the existing list. There is a list of folders for you to store your Info Message (i.e. Taxis, Weather, etc.).

.

## 6.2 Recent Calls.

【All calls】: To display the call time and duration of the answered and dialed calls. It also indicates the incoming time and dates of missed calls.

.

【Missed calls】: To display all receiving time and dates of missed calls. Press the CALL key to dial out the phone number directly..

【Received call】: To display all call time and duration of the received calls.

【Dialed calls】: To display all the phone numbers, call time and duration of the dialed calls.

【Call info】: To display call information..

Call costs:

- Last call: To inquire the fee of the last call.

- All calls: To inquire the fee of all calls.

- Clear logs: To clear the call charging records.

- Cost units: To inquire and preset the unit of call charges..

- Max cost: To inquire the top call charges and setup the max charges..

Call timers:

- Last call: To inquire call time of the last call..

- All calls: To inquire call time of the all calls..

- Clear logs: To clear the call time records..

Note: Call costs and timers are network functions. Therefore, they need to be supported by your network supplier.

## 6.3 Phone Book

This handset provides two kinds of memory: SIM card memory and handset memory. . The storage of the SIM card depends on the type of your SIM card..

Enter numbers to be stored in the standby mode and press the Left Soft Key to save. In standby mode, press the Left Soft Key to access phone book menu. Select “Complete” and access the phone book list. You can store up to 200 phone entries.

.

. 【View】.

- View details: To view the detailed information of the record. T. he detailed information includes number information, email address, etc. . The icon on the left of the first row displays the memory of the record. The display on the right indicates the storage location of record and the total number of records in the corresponding memory..

- Search by name: To search the record by a specific name..

- Edit: To edit the options.

- Delete: To delete the selected options..

- Add new name: To add a new record in phone book. .

【Add new name】: To add a new record in SIM card or handset memory..

【Settings】: To choose the storage location of the record (i.e. phone memory, SIM card, etc.).

.

【Used space】: To view the number of records stored in the SIM card and the handset. .

【Speed dial list】: To view the records saved in the speed dial list.

## 6.4 Clock

【Alarm】.

- Set alarm: To preset the alarm to ring periodically (i.e. Once only, Daily, Weekly, etc.)..

- Set auto on/off: To set the handset to switch on or off automatically..

- List alarms: To view all the preset alarm records..

- Clear all: To clear all the preset alarm records..

【Set date】: To preset the current date for clock..

【Set time】: To preset the current time of the clock..

【Set zone】: To preset your current time zone. . After successfully preset, the clock of the handset will change according to the time zone you are in..

【Clock settings】: To set up the display formats for time and date..

## 6.5 Audio Profiles

A profile is a group of settings you can use to customize the way your phone works.

【Set profile】: To preset different profiles.

- Normal: Preset various functions according to your preferences..

- Meeting: Preset to vibrate only for alerting, no ring.

- Outdoor: Ring after vibrate..

- Vehicle mode: Ring only for alerting..

- Silence: No ring and no vibration.

【Edit profile】

- Ringing volume: To adjust the ringing level..

- Earpiece volume: To preset the call volume..

- Vibrator: To preset on/off to vibrating mode.

- Keypad tones: To enable/disable touch-tone..

- Battery low warning tone: To set the warning tone on/off..

- DTMF tones: To preset on/off to Dual Tone Multiple Frequency.

- Message alert: To enable/disable the alert tone when message(s) arrive.

- Any key answer: To answer a call by pressing any key..

【Ringing tone】: To choose your preferred ring tone from 30 melodies..

## 6.6 Settings

## .

【Phone settings】: You can preset the options of the handset..

- Language: To choose from a number of languages for your handset..

- Menu style: To choose from two menu styles..

- Backlight level: To adjust the level of the backlight..

- Screen contrast: To adjust the contrast of the screen..

- Keypad light: To turn the keypad lamp on or off..

- Keyboard AutoKey: To preset the auto keypad lock to on or off. Keypad can be locked automatically when idle for 30 seconds..

- Own number: To preset the number of your handset supplied by your network provider.

【Call settings】:

- Show call number: To display your number on the calling party’s screen. This will function correctly only if your network supplier supports it.

- Minute minder: To prompt you each minute you are on the call to control your call duration. For example, you will hear the first prompt at the 50th second of your call, then, second prompt in one minute, and every other minute afterwards.

- Auto Redial: To set auto re-dial on or off.

.

.

- .

.

【Network settings】.

- Divert calls: Call divert is a network service. You have to subscribe to the service from your network supplier to use the following functions. . It can transfer a receiving call to another phone number you specified..

- . Call waiting: To set Call waiting on/off. To access this function, it must be activated in advance. This will function correctly only if your network supplier supports it.

- Select network: To select the preferred network..

  - Set Preferred Network: To select your preferred network from your list (if applicable).

  - Add new network: Refer to your new network provider for MCC code..

  - Automatic: To search the network automatically..

  - Manual search: To set the network manually..

【Security setting】: To access this menu, enter the password..

- SIM card lock: To turn on/off the SIM card lock.

- Handset lock: . Phone lock password consists of 4 -12 digits. The default password is 1234..

- Change password: To change the default password into any 4-12 digits.

- Auto keypad lock: To lock/unlock your keypad automatically when idle for more than 30 seconds..

- Handset call barring: To restrict incoming/outgoing calls..

- Bar receiving calls: To enable/disable all incoming calls..

- Bar outgoing calls: To enable/disable all or some outgoing calls..

- All calls: To restrict all incoming/outgoing calls.

- Preset field: To restrict the listed outgoing calls. The phone numbers stored here cannot be called through this handset..

- Off: To cancel the restriction of all outgoing calls..

- Network call barring:.  To bar incoming/outgoing calls to/from other networks..

- Fixed dialing:.  To enable this handset to call the specific numbers stored in this list. Your regular phone book will no longer be valid.

.

【Reset settings】: To restore settings to factory default.

6.7 Tools

Calculator.

The calculator in the handset can carry out simple and practical operations.

- * key: To enter or switch the symbols of operation, including plus, subtract, multiply, divide, etc.

- # key: To enter or switch the symbols of operation, including 4 arithmetic operators: decimal, sign of percentage, and right and left brackets..

Press the Left Soft Key to view the arithmetic result and the Right Soft Key to clear a character. Hold down the right-soft key to clear a row of character.

.

Note：Only one row is displayed on your screen. Press the DIRECTION key to scan the entire entry..

Incoming Call Guard

Call Guard lets you restrict a number or a pre-defined group of numbers. This feature makes it easy for you to manage your incoming calls by reminding you what numbers your incoming calls are from. .

- Preset numbers: To add, view, edit, delete number(s), etc. The record of calls will be saved in the missed call menu. The network responds to the caller with a voice prompt and unable to connect signal..

- Status: To inquire the status of the receiving call guard (on/off)..

## 6.8 Games

The handset provides two classic games: Russian and Bricks. Choose a game with the DIRECTION key, then, press the Left Soft Key to confirm. Follow the instructions after choosing a game for setup, game instructions and top score information..

General settings.

The general settings are used for all the games in the handset..

- Sound: To set a game’s sound on or off..

- Backlight: To set the game's backlight. Be able to set it on or off.

.

Note：In the game menu, “Resume game” menu appears only when the game history exists.

# . 7. ENTRY METHODS

# .S

Smart English Entry Method

T9 Smart English Entry Method has smart associating function, the more letters you enter, the more complete words you get. Use  key to switch capitalization. . For example: pressing , , , key will enter the word “ best ”..  Press the CALL key to search if the displayed word is not the one you want to enter..

ABC Entry Method

ABC Entry Method has no associating function. Enter a corresponding letter with each touch. . For example: Press  key to enter ‘A', press  key for ‘D', and press  key in succession to select 'A', 'B’ or ‘C’ In the Entry Method, Use  key to switch capitalization, and other keys to enter letters.

Numerical Entry Method.

Press numerical keys to enter the corresponding digits..

Symbol Entry Method.

Each numeric key represents a symbol. To enter the symbols, press the numerical key to select the desired symbol. Press DIRECTION key to scroll pages.

# 8. SAFETY AND PRECAUTIONS

- Do not touch the antenna of the handset unnecessarily; doing so may affect the sound quality of the call..

- Switch your mobile phone off when in the vicinity of chemical plants, gas stations and other explosives..

- Keep the phone away from children.

- Please use the hands-free device (optional) when driving. Park the vehicle before using the handset.

- Switch off the phone when aboard an aircraft.

- Switch your mobile phone off when in the vicinity of medical devices such as pacemakers, hearing aids, and electro-medical equipment.

- Use only approved accessories and batteries by the manufacturer..

- Your mobile phone must be installed or repaired only by qualified personnel.

- Do not charge the handset without inserting the battery first.

- Charge the phone in a well-ventilated environment.

- To avoid demagnetization, keep the handset far away from magnetic objects, such as magnetic discs or credit cards.

- Ensure to keep the handset dry. Remove the battery if leakage or erosion occurs and contact the supplier..

- Avoid using the phone outside its operating temperature range of       14°F(-10°C) to 113°F(45°C). Never leave the phone exposed to direct sunlight, or in a high humidity or dusty environment.

- Do not use liquids or strong detergents to clean the handset.

<Warning>: The manufacturer will not take any responsibility in the case of noncompliance with the above advice or improper use of the handset.

<Attention>: Dispose of used batteries in accordance with any local regulations.

# APPENDIX I : TROUBLESHOOT

| TROUBLE | POSSIBLE REASONS |
| --- | --- |
| Unable to switch on your phone | - Is the battery charged? - Is the battery installed properly? |
| Unable to charge battery | - Is the charger connected properly? - Is the battery usable? The battery performance diminishes after several years’ use. - Is the battery overly discharged? The handset comes into a slow charging mode after connected to the charger. The handset normally can be charged hours later depending on the degree of discharging. |
| Network lost | - The signal might be too weak. There may be some radio interference. Check the signal indicator for vertical line. - Is the SIM card located correctly? Is it damaged in any ways? (If damaged, please call our network supplier for replacement.) |
| Fail to preset function | - Does your network supplier support this function? |
| Unable to make calls | - Are you registered in the network? Check the screen display for the name of your network supplier. - Is Call Barring enabled? - Have you paid your charges? - Have you activated the Fixed dialing function? - Have you enabled the “Restrict my phone number” rejected by the network? |
| Unable to receive calls | - Are you registered in the network? Check the screen display for the name of your network supplier. - Have you paid your charges? - Is Call diverting enabled? - Is Call barring for incoming calls enabled? |
| PIN locked | Enter the PUK code supplied with the SIM card to unlock. |
| PIN2 locked | Enter the PUK2 code supplied with the SIM card to unlock. |
| Call meters inactive | No transferring metering pulses, please contact your network supplier. |
| SIM error | - Is the SIM card dirty? - Re-mount the SIM card. - Is the SIM damaged? (If damaged, please call your network supplier for replacement.) |

# APPENDIX II : ABBREVIATIONS AND EXPLANATIONS

| CUG | Closed User Group |  |
| --- | --- | --- |
| GSM | Global System for Mobile Communication |  |
| GPRS | General Pocket Radio Service |  |
| SMS | Short Message Service |  |
| CB | Cell Broadcast |  |
| FDN | Fixed Dialed Numbers |  |
| SIM | Subscriber Identity Module |  |
| PIN | Personal Identity Number. Supplied with SIM card. To unlock the SIM card. |  |
| PIN2 | Personal Identity Number 2. Supplied by the network provider. For special function in protected FDN supported by SIM. |  |
| PUK | Personal Unlock Code. Supplied with SIM card. To unlock the SIM card when you enter the wrong PIN code three times. You can also use it to modify PIN code. |  |
| PUK2 | Personal Unlock Code 2, supplied with a specific SIM card and it is to modify or enable a PIN2 code. |  |
| IMEI | International Mobile Equipment Identity consists of 15 digits. One mobile phone has only one IMEI. |  |
| MCC | Mobile Country Code |  |
| MNC | Mobile Network Code |  |

.

| Call Waiting | A service that allows you to receive another incoming call without interrupting the present call. It is different from multiparty calls. You can only talk to one party while another party is put on hold at the same time. |
| --- | --- |
| Call Divert | A service that allows you to divert an incoming call to a specified number. Normally calls are diverted if busy, no answer or not reachable. |
| Divert all calls | All incoming calls will be diverted by the network. |
| If busy | Divert your incoming calls while you are on another call. |
| If no answer | The network will divert your incoming call if you haven’t answered it in a specific duration when the handset rings or vibrates. |
| If not reachable | The network will divert your incoming call when the network fails to locate your handset. It could be switched off or out of the service area. |
| Roaming | It is when the handset is not used in the local registered network. It includes international and domestic roaming. In this manual, it refers to international roaming only. |

CC20040603V1.0E